Summary of Updates

- Organization and Administration

  - Revised to only 4 categories

    - Full

    - Associate

    - Guest

    - Lifetime

  - Clarified ASRI Board of Directors required for approval of all except guest.

    - Recommended for a primary member, and up to 2 alternate members, per organization.

    - More can be considered as if approved by the AFC and ASRI BoD.

  - Primary AFC member must be an employee of organization being represented.

    - Alternates can be contractors.

  - Clarified proxy voting process

    - Other organizations or Exec Sec can be nominated if needed.

    - Cannot be used for more than 2 consecutive meetings.

  - Inactive status updated

    - Initiated after 2 missed meetings

    - Only reactivated when attending a regular AFC meeting in person

  - Formal motion process update

    - Majority of membership required?

    - Specified topics that require it

    - Recording of member votes

  - New process for overturning previous formal motions

  - Specified informal motions

    - Seek any objections for minor decisions

  - Clarified online meeting process

    - For special/urgent AFC meetings

  - ListServ rules incorporated

  - New meeting conduct section

    - Confidentiality of information

    - Confirming that the AFC is a ‘closed door’ meeting

- VHF Assignments Process

  - Explain process to share co-frequency assignments (50% margin, etc.)

  - DOC table revision of altitude heights

  - Control/dispatch point regulatory requirements

  - Explain station inspection policy

  - Voice assignment calculation

  - GSARS process

  - License request process

  - Deicing

  - VDLM2 capacity management

- VHF Installation Standards

  - Terminology change

- HF Policy

  - Updated frequencies and regional info

- Member Associations, Related Industry Organizations, Federal and International Agencies

  - Consolidated into one section

  - Rewritten into small summaries

- Frequency Allocations, Coordination and Assignments

- Glossary, Two Letter State & Territory Abbreviations, Terms And Definitions

  - Updated glossary from Section 2 terms

  - Removed states list

- AFC Recommendations and Resolutions

  - Transferred to a separate AFC History Document

- AFC Composition and Member Lists

  - Updated to new data

Aeronautical Frequency Committee Manual

| Change Log |  |  |
| --- | --- | --- |
| Date | Rev. | Action/Preparer |
| May 28, 2009 | -- | Original Issue/A. Stutz |
| November 16, 2010 | A | A. Stutz |
| May 24, 2012 | B | V. Nagowski/A. Stutz |
| Oct XX, 2016 | C | A. Roy/M. Hinojosa |

FORWARD

The Aeronautical Frequency Committee (AFC) is an Aviation Spectrum Resources, Inc. (ASRI) sponsored industry committee. This Manual contains general descriptive information of both national and international organizations and agencies which are associated with aeronautical frequency management, radio regulatory and AFC activities. This Manual also contains information which is relative only to the AFC, is specifically provided as a convenient source of information for the AFC, and is intended solely for use by AFC Members and others to directly associate with its activities.

The AFC Executive Secretary will maintain this Manual in current status by issuing amendments. The amendments will be numbered and all pages of the Manual will be dated. An Amendment Record Page is included inside the cover of the Manual and should be used to reflect the status of current amendments.

Recommendations for revision of this Manual should be submitted to the Executive Secretary. This document shall be reviewed as required by the AFC.

HISTORY OF AVIATION SPECTRUM RESOURCES, INC. (ASRI)

Aeronautical Radio, Inc. (ARINC) was incorporated in 1929 as a result of public hearings on aviation held by the Federal Radio Commission (now the Federal Communications Commission). The first Commission hearing on aviation was held May 11, 1929. This was the formal beginning of aeronautical radio service. In August, 1929, a series of conferences were held by the Commission with the military and airline industry to formulate an acceptable plan for radio licensing and frequency utilization. Finally, on September 9, 1929, the Commission adopted a general order which required the airline industry and all other civil aviation operators to coordinate and consolidate frequency requirements and licensing under one central affiliated group. The purpose of this consolidation was to eliminate confusion created by each airline independently securing licenses and operating their own private radio facilities. By this time, it had already become obvious to all that a high degree of cooperation would be necessary between the airline operators if maximum efficiency in utilization of radio systems was to be achieved. As a result, on December 2, 1929, ARINC was organized and incorporated within the State of Delaware. In consonance with the company's role as the single licensee and coordinator of aeronautical radio communications for the aviation community, ARINC was formed as a normal business enterprise, but one which would be operated on a not-for-profit basis. All of the associated stock was subscribed to by the air transport operators.

On May 12, 1930, the Commission transferred to ARINC the licenses for approximately 75 high frequency (HF) ground stations which previously had been issued to the air carriers. Initially, ARINC members continued to own and staff the ground station facilities. ARINC, as the licensee, supervised the operating procedures to insure proper operation and compliance with the Commission's regulations.

Gradually, as ARINC's capabilities grew, the company was asked by most of the airlines to assume full operational responsibility for their air/ground communication requirements. ARINC's capabilities continued to expand at a level commensurate with advancements in the state of the art. Hence, the aviation community came to regard ARINC as the center of communication planning for the entire transport industry.

During the period 1930-1939, ARINC stations, staffed by airline personnel, performed airport, approach, and enroute communications functions, and were the only air/ground communications stations on the airways. This was the first time a pilot could fly along a route with complete assurance that current weather and alternate airport information was available.

It made flight following by ground controllers possible so that airline companies could make advance planning on disposition of aircraft and crews. In July, 1936, the Civil Aeronautics Authority (now the Federal Aviation Administration), with ARINC's technical assistance, established the first government air traffic control centers at Chicago, Cleveland and Newark. ARINC stations, however, continued to perform air traffic control functions.

During the same period, ARINC, in cooperation with the Civil Aeronautics Authority, played a leading role in the research and development of radio navigational aids to aircraft and instrument landing systems, and development of associated techniques and equipment operating procedures. This effort resulted in establishment of the Radio Technical Commission for Aeronautics (RTCA). The RTCA (no longer known by the former name) has subsequently become the foremost technical consulting group in the aviation community; ARINC is still an active member. ARINC also contributed to the development of numerous equipment characteristics standards and specifications, including standards for flight checking of ground facilities. ARINC developed the specifications and design for crystal controlled airborne transceivers capable of operating on a number of predetermined frequencies selected by the pilot through a switch. The equipment eliminated the requirements for pilots to make frequency adjustments. This was an important first for ARINC and was equally important to the airline industry.

Following World War II and the release of large, long range aircraft, new methods and techniques were required to satisfy aeronautical requirements for air/ground and point-to-point communications. These demands in the decade immediately following the war caused the airline industry to turn to ARINC to solve these problems. In 1948 and 1949, the volume of air/ground traffic indicated that even greater sharing of frequencies and radio operators was essential. Thus, the first of ARINC's Communication Centers was activated at Houston, Texas. Soon thereafter, similar centers were established at Anchorage, Honolulu, Los Angeles, New Orleans, Okinawa, San Francisco, San Juan, Seattle and Tokyo. In addition, wire lines simultaneously replaced the continuous wave (CW) point-to-point service, and Very High Frequency (VHF) systems began to replace HF as a medium for communicating with flight crews. By 1967, 15 operational extended-range VHF stations substantially reduced reliance upon HF systems on over water routes. The New York/Eastern Canada VHF network, which served aircraft on the North Atlantic routes, was further extended by activating a station at St. Anthony, Newfoundland.

Inauguration of the ARINC Private Line Intercity Network (PLIN) on December 1, 1969, established, in a single system, essentially all intercity circuitry utilized by the air transport industry. In 1969, the Federal Communications Commission also designated ARINC as the Advisory Frequency Coordinator for assignment of Aviation Terminal Use frequencies at all airports, a function that was transferred to the National Association for Business and Educational Radio (NABER) who ceased operations in 1995. During the same period, new equipment was installed at all ARINC gateway stations for transmission and reception of both single sideband and amplitude modulation (AM) signals. Additional HF capability was added to ARINC's stations in Hawaii and New York to augment the Major World Air Route Area (MWARA) frequency assignments in these areas. The new frequencies increased the communication capacity of all MWARA areas served by ARINC. During 1971, operational capability of the system was expanded from the initial over-ocean service at San Francisco with aircraft operating on transcontinental routes. Specially designed ARINC VHF remote radio facilities were added to six domestic locations and connected to the computerized control center at San Francisco.

Due to a reduction in air/ground contacts in 1973 and 1974, ARINC reconfigured control of its domestic VHF network system by decreasing the number of communication centers. Accordingly, air/ground operations in Los Angeles, Seattle and Denver were transferred to San Francisco, Chicago and Forth Worth in mid-year and the former were subsequently decommissioned. The Washington Communication Center was closed in October, and the associated air/ground operation was transferred to New York. The San Juan Communications Center was closed in 1989 and Honolulu in 1994. With these closures, control of the air/ground service is now exercised through ARINC Communications Centers at New York and San Francisco. Early in 1974, the last remaining manual message switching operation was eliminated when this function was mechanized at the Honolulu Communication Center.

In 1978, the Aircraft Communications Addressing and Reporting System (ACARS), was added to the services provided by ARINC to the airline community. ACARS is the air/ground/air communications service that provides for the exchange of digital information between the ground and ACARS-equipped aircraft. Today, this service is available throughout the continental United States, Hawaii, Alaska, Puerto Rico, Mexico, Canada, South America, Europe, and Asia.

The foregoing is a very brief historical account of ARINC's growth and facilities. This growth was correlated to expansion of the U.S. airline industry. Similarly, airline requirements for communications grew at a commensurate pace, and technology and communication-electronic systems became more complex. Consequently, the Aeronautical Frequency Committee (AFC) was established in 1949 to develop and maintain a single airline voice in frequency management activities. In 1955, the Air Lines Communications Advisory Council (ALCAC) was established, and the AFC became a standing committee of the Council. The AFC remained a standing committee of ALCAC until December, 1974. At that time the ARINC Board of Directors made it directly responsible to the Board and recommended expansion of its membership, insofar as practicable, in order to be representative of all users of ARINC's radio services.

In order for ARINC to proceed with obtaining outside financial support, ARINC petitioned the Federal Communications Commission (FCC) in September 2004 with a request for Declaratory Ruling that would alter the manner in which VHF and HF spectrum used in aeronautical enroute service was managed. This request would not only allow the change in spectrum management but would also enable the restructuring of ARINC. In March 2005, the FCC granted ARINC’s request to move to a new entity under aviation industry governance and control the management of VHF and HF frequency spectrum used to provide the aeronautical enroute service in accordance with Part 87 of the FCC rules that was currently licensed to ARINC. The ARINC Board of Directors asked Management to proceed with creating a new entity. As a result, on January 1, 2006, Aviation Spectrum Resources, Inc (ASRI) was organized and incorporated within the State of Delaware.  A separate ASRI Board of Directors consisting of airline management staff was formed to provide guidance and oversight to the new company.

Table of Contents

# Organization and Administration

## RESOLUTION

Action by Written Consent of

Board of Directors of

Aviation Spectrum Resources, Incorporated

December 31, 2005

“The undersigned, being the sole member of the Board of Directors (the “Board”) of Aviation Spectrum Resources, Incorporated, a Delaware corporation (the “Corporation”), in lieu of holding a special meeting of the Board, hereby takes the following actions and adopts the following resolutions by unanimous written consent, pursuant to Section 141(f) of the Delaware General Corporation Law (the “DGCL”): Appointment of Directors, Appointment of Officers, and General Resolutions.

This Action by Written Consent of the Board of Directors of Aviation Spectrum Resources, Incorporated is effective as of the date first set forth above,” Signed John M. Belcher

Adopted by the ARINC Board of Directors

October 20, 1999

"WHEREAS, it is the desire of the Board of Directors of Aeronautical Radio, Inc. to call upon the expertise and assistance of the users of ARINC services in the development of plans and policies with regard to the use and regulation of the radio frequency spectrum, and believes that it would be beneficial to the Board and to the users of ARINC services were the membership of the Aeronautical Frequency Committee expanded to be as representative as practicable of all aircraft operators using ARINC's air-ground radio services;

BE IT THEREFORE RESOLVED, that the Aeronautical Frequency Committee shall be directly responsible to the Board of Directors; that the Terms of Reference attached are adopted for the Aeronautical Frequency Committee; and that the Aeronautical Frequency Committee is requested to make such recommendations and suggestions as it may deem appropriate for consideration by the Board of Directors."

## TERMS OF REFERENCE

### Purpose

The Aeronautical Frequency Committee (AFC) shall be an ASRI sponsored industry committee providing assistance in the formulation of industry recommendations and policies relating to the allocation, assignment and use of the VHF and HF radio frequency spectrum.  The AFC also considers regulatory matters pertaining to all spectrum used by commercial aviation operations, at both domestic and international levels.

In fulfilling its purpose, the Aeronautical Frequency Committee (AFC) shall study aeronautical and land mobile systems concepts and recommend to the ASRI Board of Directors the assignment, use and control of radio frequencies related thereto.  The AFC shall also maintain a constant awareness of the status of regulatory matters pertaining to the use of the radio frequency spectrum, and recommend, where necessary, appropriate action to the ASRI Board of Directors.

### Membership

Membership shall be comprised of major and regional US based commercial aircraft operators, air/ground service providers and organizations representing aircraft operators in business and private general aviation. The constituency of organizational memberships, and affiliations among airlines should be considered to maximize representation of the US commercial aviation industry, while also minimizing duplication of representation.

It is recommended that individuals nominated for membership in the Aeronautical Frequency Committee be experienced in aeronautical communications, radio frequency planning, and communications regulatory matters.

The membership of AFC shall be composed of named Full Members and their alternates (voting), , associate (non-voting) members, lifetime honorary (non-voting) members, and , and guest (non-voting) members.  Additionally, ASRI shall designate one voting member from the ASRI staff. The AFC will review its membership regularly, and make recommendations for any additions or deletions to the membership to the ASRI Board of Directors.

### Officers and their Duties

#### Chairman

A Chairman, whose term shall be one calendar year, shall be elevated from the office of Vice Chairman from the preceding year, or elected each year by the committee from among its members should that office have been vacant.  The Chairman may not immediately succeed themselves in office except that, if the elected Chairman should vacate the post prior to the expiration of his/her term, the Vice Chairman shall automatically assume the office of Chairman, but shall remain eligible to fulfill the office of Chairman during the following term.

It is the duty of the Chairman to call the meeting to order at the appointed time; to preside at all the meetings; to announce the business before the Committee in its proper order; to state and put all questions properly brought before the Committee; to preserve order and decorum; and to decide all questions of order (subject to an appeal).

#### Vice Chairman

A Vice Chairman, whose term shall be one calendar year, shall be elected each year by the committee from among its members.  In the event of the absence of the Chairman, The Vice Chairman will assume all of the Chairman's authority and duties.  After serving one year, the Vice Chairman shall normally serve as Chairman for the following calendar year commencing January 1 upon approval of the AFC membership.

#### Executive Secretary

An Executive Secretary, not a voting member of the committee, shall be appointed by ASRI and shall receive, maintain and distribute correspondence, bulletins, documents, minutes of meetings and other written material of the committee.  They shall also maintain technical liaison with other organizations (e.g., ATA, IATA, etc.) and advise the AFC Chairman of AFC policies, previous committee decisions, and matters requiring committee attention.

### Election of Officers

Officers for a succeeding year shall be elected during the last Committee meeting of the current calendar year; i.e. usually the Fall Meeting.  Full members (primary or alternate), and associate members may be elected to become Vice Chairman, and eventually Chairman, of the AFC.

Once all nominees for Vice Chairman are collected by the Executive Secretary, an open vote will be carried out between the AFC full members.  The nominee with the most votes will be elected.  In event of a tie-breaker, the vote will be recast.

#### Vacancies

In the event the Chairman's position becomes permanently vacant, the Vice Chairman shall automatically assume Chairmanship of the AFC for the remainder of the calendar year, and shall continue to serve as Chairman the following year.  If a situation dictates the Vice Chairman assume the chairmanship, or if the Vice Chairman's position becomes vacant, a new Vice Chairman shall be elected to serve out that term, and that of the following year

#### Temporary Chairman

In the event the Chairman and Vice Chairman are unavoidably absent from a meeting, the Committee shall elect a temporary Chairman for its first agenda item at the affected meeting.  The Executive Secretary shall preside with full rights as Chairman until a temporary Chairman has been elected.

### Modification of Committee Terms of Reference

Any proposed amendment to the Committee Terms of Reference shall require a favorable two-thirds vote of all full active members, and a previous notice that a proposed amendment is to be considered during a specific meeting.  The notice shall be distributed to all Committee Members at least three weeks prior to the date the proposal is to be voted upon.  All changes to the Terms of Reference are subject to approval by the ASRI Board of Directors.

## MEMBERSHIP CATEGORIES

### Full Membership (voting)

AFC Full Membership is available to organizations with a significant usage of ASRI VHF or HF licenses across the US.

Full membership status (known as a ‘Full Member’) of the AFC allows an aviation organization to cast a single vote through its named primary or alternate members on its behalf, on matters of AFC policy, actions, or procedure (known as a ‘Formal Motion’) by the committee.

Additionally, any motion proposing a policy, action, or record, must be proposed or seconded by at least one Full Member organization to initiate an AFC formal motion.  An organization with full membership is allowed one named primary member, and up to two named alternates, any of which can vote depending on availability at AFC meetings.  Should the AFC organizations of two or more existing Full Members merge or combine, the voting rights will be combined once the AFC is notified of the merger’s completion.

The named primary AFC Full Member must be a direct employee of the organization being represented.  Alternate members may be additional employees, nominated contractors, or a combination of both.  However, nominated contractors must be recommended by an AFC Full Member organization in accordance with paragraphs 1.2.2 and 1.3.1 of the AFC Manual.  Contractors will be on probation for two attended meetings before the AFC votes to recommend them to the ASRI Board of Directors for actual membership, including the ability to vote. Additionally, an individual contractor will only be eligible to represent one AFC Member; any changes to representation will be treated as if the contractor was being proposed as a new nomination.

#### Proxy voting on behalf of full member

An organization with full membership may nominate a single proxy member on a temporary basis should it be required.  A proxy member represents the nominating organization at the meeting and inherits the full meeting rights of that member.  The nominated proxy member can only be an employee from the same organization, an existing AFC member from another organization, or the AFC Executive Secretary.   A formal written request for a proxy shall be provided to the Executive Secretary prior to commencement of the meeting at which it is to be exercised.  The Executive Secretary shall inform the Committee Members of the proxy authorization immediately after the Chairman calls the meeting to order.  A separate proxy request should be submitted for each meeting it is required, and cannot be used for more than two consecutive meetings by the same AFC organization.

Any AFC Member who nominates a proxy for a meeting, shall be responsible for ensuring the individual complies with all AFC policies.

#### Inactive status of full members

Any AFC organization who permanently nominated representatives fail to attend two consecutive regularly scheduled AFC meetings in-person will be considered as inactive.  Inactive members will be notified by the AFC Executive Secretary, and will not count towards calculating the total number of AFC members for a Quorum, be ineligible to vote in formal motions at any AFC meeting, and be unable to nominate a proxy.  An inactive full member is automatically reinstated to active membership upon attendance in-person, or represented by an approved alternate member, at an AFC meeting.  Attendance of an Irregular or Online AFC meeting, SWG, dialing into any AFC meeting remotely, other Special meetings, or proxy attendance, will not be considered attendance in determination of inactive status.

### Associate Membership (Non-Voting)

Representatives of the Federal Aviation Administration, aviation trade associations, manufacturers, and other aviation industry representatives with a long-term and direct involvement in AFC business may be nominated to the AFC as Associate Members. Up to three named employees from each organization can be nominated as AFC Associate Members.  Such nomination shall be forwarded to the ASRI Board of Directors for confirmation of appointment.

Associate Members shall not be entitled to vote at meetings. Additionally, the AFC may hold closed meetings or sessions to which some or all Associate Members are not invited, or from which certain Associate Members may be excluded.

### Guests (Non-Voting)

Guest status is intended for aviation organizations in communications matters that can provide a high level of expertise to AFC Members, but would not normally be considered for Associate membership.  Their participation with AFC activities may be solicited when appropriate, and can include guest speakers from relevant organizations, such as non-AFC airlines, service providers, equipment manufacturers, etc.  The AFC Chairman has the authority to invite or approve requests for such representatives to be present at portions of the AFC Meetings where the expertise of these organizations is required to address specific agenda items.  Any observer invited by a full AFC member as a co-worker, or prospective member, for the purpose of introduction and familiarization with the AFC, will remain as a non-voting guest unless a proxy request has been received before the meeting.

The AFC may hold closed meetings or sessions to which some or all guests are not invited, or from which certain guests may be excluded.

### Lifetime Honorary Membership (Non-Voting)

The Committee may, in recognition of the contributions of former members, confer a "Lifetime Honorary Membership" upon those members who have contributed to the achievement of more effective utilization of the radio frequency spectrum while serving as a member of the Committee.  An appropriate certificate will be presented to former Members accorded the "Lifetime Honorary Membership."  Lifetime Honorary members will be considered to have guest status if attending AFC meetings.

### Requests for New, or Changes to Existing, Members

Requests for new, or changes to existing, Full, or Associate Membership must be submitted in a formal letter on company letterhead to the AFC Executive Secretary naming all nominated personnel to the AFC who will be representing the organization (including reconfirming the existing personnel who are already on the committee).  The formal letter must be from a higher level employee of the organization to be represented, and who is responsible for that agency's interest in air-ground communications.  Nominations for lifetime honorary membership may be made by a formal motion during an AFC meeting by an existing AFC Full Member.

The AFC Executive Secretary will present all received membership requests at the next full AFC meeting for a formal vote by the existing AFC membership, then forward the AFC approved nominations to the ASRI Board of Directors for ratification.  Voting rights for named full members will only be ratified once the ASRI Board of Directors has formally approved their membership.

Any new AFC Full Members nominated to the AFC from an organization not already on the committee, must be present at the AFC meeting where their membership is voted on.  If they are not present, then the vote to confirm them will be postponed until the next meeting they are available.

Membership removal?

### Past Chairmen

A full list of previous AFC Chairmen are listed in Appendix A.

### ASRI Board of Directors

The organizations forming the ASRI Board of Directors are listed in Appendix B

### Current AFC membership

The list of AFC organizations and their representatives can be found on the AFC members site https://www.asri.aero/afc-membership/afc-members-only/ .

## Conduct of Meetings

### General

Reports of AFC meetings will be issued to its members and to other interested parties.  Industry recommendations and positions developed by the AFC will be referred to the ASRI Board of Directors.  Upon approval by the Board, ASRI shall be responsible for the prosecution of such recommendations and positions before the applicable government agencies and in such other forums as deemed appropriate.

The AFC Executive Secretary shall keep the ASRI Board of Directors and applicable users informed on schedules of meetings, projects instituted and terminated and other important significant actions or changes.

Administrative, budgetary and regulatory aspects of AFC activities will be subject to review and supervision by ASRI.

### Regular Scheduled Meetings

The AFC shall conduct three regular scheduled meetings a year in person, at locations to be determined by the approved AFC membership.  Meeting locations will be selected based upon geographical region, taking into account prior meeting sites and member requests.  Each meeting will be provisionally selected two meetings in advance, and finalized the meeting before.  Meeting schedules will be subject to review and approval by ASRI Management dependent on logistical and financial considerations.

### Special or Online AFC Meetings

Should it be formally requested by the membership to the AFC Chairman, additional AFC meetings may be held either in person (a Special AFC meeting), or online.  These meeting will follow all the normal AFC rules for conduct and procedure, except for the rules addressing inactive status as specified in Section 1.3.1.

Any requests for additional meetings to the AFC Chairman shall include the principle objectives for the meeting, and a proposed agenda.  All requests should be submitted in ample time to allow consideration by the AFC Chairman, coordination with the AFC membership for attendance, and organization or venues and material by the AFC Executive Secretary.

All Special or Online Meetings must be approved by the AFC Chairman, taking into account the advice of the AFC Executive Secretary on logistical and financial considerations, and also the availability of the AFC membership wishing to attend the meeting.

### Quorum

For all Regular, Special or Online AFC meetings, a quorum shall be confirmed when 50% or more of the active Full Members of the committee are in attendance, including proxies. Inactive members will not be counted in determining whether or not a quorum exists.

Should a quorum not be present, the meeting may continue.  However, any formal motions on AFC policy, action or procedure will be held until the end of the meeting, to be distributed by the Executive Secretary by email in accordance with the remote voting rules of section 1.4.1.5.  Inactive members will not be counted in determining whether or not a quorum exists.

### Voting Eligibility and Methods

Each AFC organization with full membership shall be entitled to one vote by any one of its named members, or proxies, on all formal and informal motions.  All votes may be cast in person by the primary member, by the alternate member, or by proxy.

Voting methods are split into formal and informal motions, to ensure meeting can proceed as easily as possible, while also maintaining formal approval for significant decisions.

--

#### Formal Motions

Formal motions are initiated for AFC matters of policy, action, or record, and require both a proposer and seconder, before the AFC Chairman puts it to a vote.  All formal motions require a majority vote of all active AFC Full Members in order to be approved, and appropriate text should be clarified before any motion is voted on (either verbally for simple motions, or in writing for more complex issues).  Items requiring formal motions include:

- AFC membership changes to be recommended to the ASRI board of directors

- AFC policy changes or decisions

- AFC action plans to be implemented by ASRI

- AFC messages or positions to be formally represented to external entities

- Election of AFC Officers to full and temporary positions

- Approval of the AFC meeting minutes

- Meeting commencement and adjournment

Once approved, all Formal Motions become official AFC positions for the AFC Membership, and will be adopted with immediate effect unless ASRI Board of Directors approval is required.

#### Informal Motions

Informal motions cover procedures and conduct during AFC meetings, such as:

- Meeting agenda and structure

- Meeting locations and dates

- Administrative issues

Informal motions can be made by both AFC Full and Associate Members, and will take the form of a request for any objections to a defined proposal.  If no objections are received, then the Informal Motion will be taken as agreed.

Should any objections be received, and these cannot be easily addressed by the proposer with some minor modification, then the Informal Motion shall need to be made into a Formal Motion to proceed.

#### Urgent Issues Requiring Votes by Email

When necessary the Executive Secretary, at the direction of the AFC Chairman, may disseminate urgent issues with a request for a vote by email .  In this event, a -majority vote of all active AFC Full Members shall be required to adopt an issue or position.  The Executive Secretary shall inform the Committee of the results of the vote, including which members voted and for what.

#### Recording Committee Votes

The minutes of meetings shall indicate the proposer and seconder of Formal Motions, and whether a decision was unanimously supported, approved by majority vote, or lost.  The total number of votes for, against, and abstentions shall be reflected in the minutes.  Any AFC Full Member has a right to request their specific vote be recorded in the minutes.

There is no requirement to record Informal Motions in the meeting minutes, other than the outcome and noting that it was agreed.

### Overturning previous formal motions

Any motion to overturn a previously approved formal motion as specified in Section 1.4.5.1 shall require a higher level of approval.  However, the proposal must be submitted in writing to the AFC Chairman and Executive Secretary at least 14 calendar days before any potential vote, to allow the AFC membership time to consider the proposal.  Any submissions received after this deadline may be considered by the AFC only if no objections are received.

When being voted upon, at least two-thirds of all active AFC Full Members must support the motion for it to pass.  Once approved at the AFC level, any decision to overturn an existing formal motion must be approved by the ASRI Board of Directors.

### Special Working Groups

Special Working Groups (SWG) may be established to address specific projects and issues which require development of an Industry position, policy, or recommendation.  Membership in a SWG shall be open to all members of the Committee.  Conveners of a SWG shall present aregular report at Committee meetings.  All Industry positions, policies, and recommendations developed by a SWG are subject to approval by the full Committee prior to implementation or acceptance as an Industry position.  SWGs will be disestablished after the Committee approves the output  of the Group, and agrees its work has been completed.  The Committee may reactivate a SWG if deemed necessary.

### Minutes

The minutes of Committee meetings shall be the official record of decisions, including all business transacted, activities undertaken, plans projected, etc.  The minutes shall be signed by the Executive Secretary and sent to Committee Members or posted on the AFC Website prior to the next regularly scheduled meeting.  The minutes may be corrected whenever an error is noticed regardless of the time that has elapsed. However, after their adoption, when it is too late to reconsider the vote, a two-thirds vote for amendment shall be required unless previous notice of a proposed amendment has been given; then only a majority vote is required.  The minutes of meetings shall include the following:

a.	Whether it was a Regular Scheduled, Special, or Online AFC meeting.

b.    The name of the presiding Chairman.

c.	Whether the minutes of the previous meeting were approved.

d.	The names of all Members (or proxy) present and absent.

e.	All Formal Motions, whether adopted or lost, including the number of votes placed for, against, and abstained.  The complete text of all adopted Recommendations and Resolutions shall be recorded in the minutes.

f.	The names of persons proposing and seconding Formal Motions.

g.	Points of order and appeals, and whether sustained or lost.

h.	Summarized reports of Special Working Groups, unless written reports are appended to the minutes.

i.	All appointments to Special Working Groups, elected officers, etc.

### Order of Business

The order of business for a regularly scheduled Committee Meeting shall be as follows:

a.	Approval of the Agenda

b.	Consideration of and approval of the Minutes of the previous meeting

c.	Reports of Related Activities

d.	Status of Systems in Use

e.	Reports from Special Committees

f.	Unfinished Business (as previously announced in a formal agenda)

g.	New Business

h.	Consideration of Dates and Locations for the Next Meetings

- Member Roundtable

### Conduct at Meetings

Given the presence of multiple aviation organizations in the same meeting, any presentations or statements made in an AFC meeting should not

- Involve marketing or sales messages.

- Anti-trust rules

At the discretion of the AFC Chairman, any member deemed to be breaking this requirement may be asked to temporarily leave the meeting.

The AFC meetings are considered a ‘closed door’ meeting environment, allowing for the free sharing of information that would not normally be publically available.  All information presented within the AFC meetings and provided on the AFC Members website is confidential in nature, and should not be realized outside the AFC membership without approval of the originating author.

Any views expressed within the meeting are not considered representative of the AFC membership unless approved in the minutes, or positions approved by formal vote within the meeting.  Any views expressed outside the AFC that do not comply with this requirement will be contested by the committee.

### ListServ

The AFC ListServ email distribution list allows any AFC member to send relevant information, documents or updates to the whole AFC membership.  It is primarily intended for working level coordination of the AFC at an informal level, and users are encouraged to communicate on common issues or questions for the benefit of the AFC membership.

The ListServ distribution system is private, and only the formally approved AFC members can receive or send emails using the system.  As per normal AFC meetings, any presentations or statements made on the AFC ListServ should not involve marketing or sales messages.  At the discretion of the AFC Chairman, any member deemed to be breaking this requirement may be temporarily banned from the ListServ.

Additionally, any AFC Member email address that appears to be ‘spamming’ the ListServ, may be temporarily banned by the AFC Executive Secretary without warning for the benefit of the entire membership until the issue is resolved.

The AFC meetings are considered a ‘closed door’ meeting environment, allowing for the free sharing of information that would not normally be publically available.  All information presented within the AFC meetings and provided on the AFC Members website is confidential in nature, and should not be realized outside the AFC membership without approval of the originating author.

Any views expressed within the meeting are not considered representative of the AFC membership unless approved in the minutes, or positions approved by formal vote within the meeting.  Any views expressed outside the AFC that do not comply with this requirement will be contested by the committee.

# Aeronautical Industry Operational VHF Policy

## INTRODUCTION

In the United States, the Federal Communications Commission (FCC) has designated a portion of the VHF Aeronautical Mobile (R) band as the Aeronautical Enroute Service (AES), for use by aircraft operators to fulfill their requirements for operational control communications.  The AES sub-bands, 128.825 to 132.0 MHz, and 136.500 to 136.975 MHz, have been designated in the U.S. for AES purposes, using either 8.33 kHz or 25 kHz channel spacing. The last four 25 kHz channels of the sub-band 136.500 to 136.975, are designated for use in internationally compatible data systems. The Aeronautical Industry VHF Policy addresses the industry criteria for assignment, sharing, and use of these channels.  Under this Policy, all channel assignments will be made by ASRI pursuant to the guidelines set forth herein.

## Regulatory and Operational Requirements

The FCC requires that ASRI, as licensee of the stations referred to in this manual, must control the use and operation of each such radio station. Any licensee assigned a frequency must be able to prove the legality of the operations in all respects as well as the nondiscriminatory nature of the service rendered to all users who have made necessary prior arrangements. Therefore, all operating personnel at ASRI radio stations have well-defined, specific responsibilities to ASRI, the licensee, with respect to matters related to compliance with applicable Federal acts, rules, and regulations.

Because radio frequencies available to the Aviation Services are extremely limited, it is mandatory that all users obtain the maximum use of the available frequencies. This includes shared use of frequencies and constant application of improved operating techniques and improved equipment.

Companies that own or lease radio station properties from ASRI, in addition to those providing certain services, must assume obligations to ASRI. This is required because ASRI's authorization from the FCC is dependent on ASRI's control over the physical properties of the radio station. This arrangement between a Company and ASRI for use of AES frequencies is covered by a contract or ASRI Service Agreement.

The Policy addresses general conditions in channel assignments, permissible communications, specific assignment criteria for voice and data systems, specialized terms and categories of service, and the U.S. - Canadian Agreement on the VHF Aeronautical Mobile spectrum.

## DEFINITIONS AND EXPLANATORY REMARKS

### General Information

In the Aeronautical Mobile (R) Service (AM(R)S) sub-bands 128.825-132.0 and 136.5-136.975 MHz, certain voice and data operations are authorized.  Voice operations are restricted solely to AOC communications.  Data communications may consist of either Safety (AOC and Air Traffic Services [ATS]) communications or aeronautical administrative communications (AAC).  AAC is allowed on a secondary, non-interference basis and is authorized for the data mode only.

### Radio Station License

Section 30l of the Communications Act of 1934, as amended, provides that no radio station may be operated except under, and in accordance with, a station authorization granted by the FCC.

### Terms

Additional terms available in Chapter 9	)

#### ACARS

The character oriented Aircraft Communications Addressing and Reporting System for data communications.

#### Adjacent Channel

A channel whose center frequency is spaced from another by one channel increment, within the same service volume.  The current channels spacing is either 8.33 kHz or 25 kHz.

#### Adjacent Signal

The frequency of the channel next nearest in frequency to another co-located channel, without regard to the number of unassigned frequencies in between.

#### Aeronautical Operational Control (AOC) Communications

Communication service within the Aeronautical Enroute Service (AES) between an aircraft and the aircraft's operating agency.  It includes communications to and from an aircraft when the aircraft is in flight status.  The aircraft operating agency refers to the dispatch, maintenance, scheduling, operating agency headquarters or others involved in the operation of a flight.  Communication may be one-way or two-way between personnel, computers or other storage or readout devices.  The communication may be conducted directly between the pilot and company offices or through a third-party radio operator.  These communications can be classified as enroute Extreme High Level (EHL), Super High Level (SHL), High Level (HL) or Mid-Level (ML), terminal or in-range Low Level (LL), Helicopter (HO), and ground Ramp (RT) communications (see Tables and Figures 2-1 and 2-2).

Voice operations are restricted solely to AOC communications.

#### Air Traffic Services (ATS) Communications

Encompasses Air Traffic Control (ATC), Automatic Weather Observation Systems (AWOS), Ramp Control (RC), and numerous other functions.  While ATS functions are normally provided by the dedicated ATS frequencies set aside for air traffic services, ATC and RC functions may be conducted on AES frequencies due to their relationship to safety of flight.

#### Co-located

Any two or more ground stations whose antennas are located within 80 feet of each other are considered to be co-located.

#### Common-user system

A radio communications network whose operator provides service to any aircraft operator who enters a cooperative agreement with a Service Provider for provision of those services, and which is, in fact, regularly serving more than two independent aircraft operators.

#### Cosite

Any two or more ground stations whose antennas are located within 1 nautical mile of each other are considered to be cosite.

#### Control Point

A Control Point is the location at which the radio operator responsible for assuring proper operation of a transmitter is located.  Radio Stations in the Aviation Services must be provided with a control point at the location of the transmitting equipment, unless otherwise specifically authorized.  Additional control points at locations other than the transmitter location may be authorized.  All such additional control points shall be shown on the station authorization (license).  A control point must meet the following conditions:

- The location must be under the control and supervision of the licensee;

- It must be provided with monitoring facilities and a means to render the transmitter inoperative;

- It must be equipped to provide the operator the ability to aurally monitor transmissions originating at dispatch points under his supervision and equipment must be arranged to provide the operator with the ability to disconnect any or all dispatch point circuits from the transmitter.

- It must be equipped to provide the operator with a visual indication that the transmitter is operating.

#### Dispatch Point

A dispatch point is a location from which radio communications may be conducted, which is under the direct supervision of a Control Point.  Dispatch Points need not be shown on the FCC license.

#### Enroute Communications

Communication, other than "Terminal (In-Range)", between an aircraft and its operating agency when the aircraft is at, or above 10,000 feet.

#### Co-channel Exclusion Zone

For a given frequency, the area around a data link ground station within which no voice stations or other independently operated co-channel data link stations will be authorized or the area around a voice station within which no data link stations will be authorized. The radius of the exclusion zone is determined from Table 2-1.

#### Flight Status

Flight status is normally defined as beginning when the flight crew enters the flight deck of the aircraft for a particular flight and ending when the flight crew leaves the flight deck at the completion of that flight.  In addition, when an aircraft is taxied by authorized ground personnel it is also considered to be in flight status.

#### Frequency vs. Channel

The term "channel" applies to a communications path which supports one contact at a time within a given geographical area, sharing a common radio frequency.  Thus, in a given coverage area, several users may share a "channel".  The term "frequency" is used when referring to a spectral band, which can be used to support one or more "channels" in different geographical areas.

#### Ground Communications

Communication between ground personnel and an aircraft on the ground when in flight status.  These communications are typically carried on without a third party radio operator.  This includes, for example, maintenance, ramp control, etc.

Essential communications to or from an aircraft required for servicing the aircraft when not in flight status are permitted for authorized ground personnel when other means of communication are not readily available.  This will permit the ground crew servicing the parked aircraft to communicate with other support personnel relative to the needs of the aircraft and to perform checks of the communications equipment.

#### Network

A system of two or more stations connected by a common communications circuit and operated from one or more control points

#### Service Volume

The protected region within which communications service is offered via a ground station.  For all line-of-sight AES services, it is a segment of a sphere, concentric with the earth, defined by the intersection of a plane tangent to the earth at the point where the ground station is located, and a sphere of radius equal to 4/3 of the earth's radius plus the maximum altitude of aircraft to be served by the ground station.  For RT stations, a transmitter height of 50 feet is assumed.

FIGURE 2-1

#### Terminal (In-Range) Communications

Communication between an aircraft and the arrival or departure ground personnel.  This communication is typically carried on without a third-party radio operator.

#### VHF Digital Link Mode 2 (VDLM2)

The internationally standardized VHF digital link system, as defined by ICAO Annex 10 SARPs Chapter 6, Section 6.4.3.

## CHANNEL ASSIGNMENTS AND CONDITIONS

### Application

Applications for assignment to Aeronautical Enroute channels will be reviewed by ASRI Frequency Management for completeness and for conformance with this Policy.  A full list of parameters is provided in Appendix X, noting that some parameters are mandatory before an assignment is made.  Applications not meeting this requirement will be returned to the applicant as incomplete.  Applicants will then be assigned to channels subject to availability and shared use.  The method of meeting the identified communication requirement will vary, depending upon the channel congestion in the requested service area.  The procedures detailed below will be used to determine how requirements will be satisfied.

### Justification of Requirements

Applicants will be required to determine and justify their request for an assignment in accordance with the criteria contained in Sections 2.6 or 2.7 of this policy.  Ground handling companies that are requesting frequencies in support of other air carriers, will justify their requests for one or more frequencies with letters of justification from the air carriers they support. Requests which are not accompanied by such justification will be returned to the applicant as incomplete.

### Conditions of Assignment

Due to regulatory requirements and constantly changing demands placed on shared spectrum resources, any assignment may be revoked or modified.  To the greatest extent practicable, however, assignment of an operating entity to one or more channels will remain without alteration so long as the following conditions are satisfied:

- Communications services are in fact provided under the assignment,

- Operations are conducted in accordance with the provisions of the ASRI Rules and Regulations for Ground Radio Stations operating in the Aviation Services

- The assignment continues to meet the criteria of Section 2.6 or 2.7 of this policy.

The assignment criteria in Sections 2.6 and 2.7 will be reviewed and updated periodically to reflect operating experience, technological advancements, and revisions to industry standards.

### Guidelines for Assignments to Channels

FCC Rules do not permit Private Aeronautical Enroute Service ground stations.  "Service must be provided to any aircraft station licensee who makes cooperative arrangements for the operation, maintenance and liability of the stations which are to furnish enroute service."

Assignments to channels will be made on a case-by-case basis in the service area under consideration.  ASRI will select the appropriate channel, sharing channels wherever possible, after due consultation with the aircraft operating agencies concerned, considering the channel loading and nature of the operations involved.  Where sharing is deemed appropriate and necessary, ASRI will insure that all concerned users are informed.

To reduce the potential for interference and permit the implementation of the maximum possible number of channels at terminal areas, it is recommended that transmitters located within the confines of airports use the minimum Effective Radiated Power (ERP) needed for adequate communications (i.e., normally 10 watts ERP or less).  However, such systems should be limited to no more than 25 watts ERP.  The system needs will be reviewed by ASRI at the time of application.

In the assignment to channels, due consideration will be given to the protection of established operations from interference to and from other operations.  Determination of whether interference protection between operators employing the same frequency is to be provided shall be based on consideration of channel loading.  In the event harmful interference is caused by a new service to an established facility, reduction of interference to a non-harmful level by engineering techniques or facility relocation is the responsibility of the new user.

If the applicant has a preference as to a frequency using either 8.33 kHz or 25 kHz channel spacing, the applicant will include this preference on the ASRI coordination form.  ASRI will endeavor to accommodate the applicant's preference.  Where the applicant has no preference, ASRI will select the best frequency to be used.

Voice and data communications will be accommodated on separate frequencies unless, as a rare exception, coordinated for air-to-air line-of-sight separation.

Table 2-1 and Figure 2-2 show the separation required between co-channel ground stations in order that aircraft radios operated at the extremes of their respective service volumes will not be within line-of-sight of each other.  Ground stations are separated by twice the sum of the radii of the two service volumes.  These distances represent the ICAO recommended separation required between co-channel ground stations in order that aircraft operating at their nominal altitudes and at the limits of the functional service range of their respective ground stations will not cause harmful interference to one another.

Table 2-2 and Figure 2-3 show the separation required between co-channel voice ground stations for assignment of channels where none are available that would meet the criteria of Table 2-1.  These separations provide only air-ground-air protection, and do not consider air-to-air protection.  Thus, a ground station would not hear aircraft attempting to communicate with the adjacent service volume, but that aircraft's communications would be heard by approximately 50% of the aircraft operating in the adjacent service volume.  This will greatly reduce channel capacity, since 50% of the air/ground communications in one service volume constitutes traffic, during which the channel is not available, in as many as six adjacent ones.

Data ground stations must be separated from co-channel analog voice ground stations by the full ICAO recommended distances in order to prevent air-to-air interference, and to comply with FCC Rules regarding prohibition of data on channels also used for voice communications, except where the two are multiplexed.

While separation of voice ground stations in accordance with ICAO/FAA standards is desirable, the shortage of AES frequencies has necessitated greatly reduced geographic separation.  Protection from air-to-air interference must be reduced to approximately 50% in order to accommodate current spectrum demand.  Therefore, the ground station geographical separation indicated in Table 2-2 is presently used on a case-by-case basis, as determined by ASRI Frequency Management, for AES co-channel assignments.

Interference mechanisms

Both tables were derived using the formula for the distances to the radio horizon from a station in an aircraft.  This formula is given below.

D= K       h

Where	D = distance to horizon in nautical miles

h = height of aircraft station in feet

K = 1.23 for an effective earth's radius of 4/3 of the actual radius

TABLE 2-1

ICAO RECOMMENDED

GROUND STATION SEPARATION

FOR LINE-OF-SIGHT SERVICE VOLUMES

(NEAREST 5 NAUTICAL MILES)

|  |  | RT GND | HO 2000 | LL 15000 | ML 24000 | HL 45000 | SHL 70000 | EHL 300000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EHL | 300000 |  |  |  |  |  |  |  |
| SHL | 70000 | 670 | 760 | 950 | 1030 | 1175 | 1300 |  |
| HL | 45000 | 540 | 630 | 825 | 905 | 1045 | 1175 |  |
| ML | 24000 | 400 | 490 | 580 | 760 | 905 | 1030 |  |
| LL | 15000 | 320 | 410 | 605 | 580 | 825 | 950 |  |
| HO | 2000 | 125 | 220 | 410 | 490 | 630 | 760 |  |
| RT | GND | 35 | 125 | 320 | 400 | 540 | 670 |  |

FIGURE 2-2

FIGURE 2-2

TABLE 2-2 and FIGURE 2-3

GROUND STATION SEPARATION FOR

AIR-GROUND-AIR PROTECTION ONLY

(50% or Less Air-Air Protection)

FOR LINE-OF-SIGHT SERVICE VOLUMES

(NEAREST 5 NAUTICAL MILES)

|  |  | RT GND | HO 2000 | LL 15000 | ML 24000 | HL 45000 | SHL 70000 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SHL | 70000 | 335 | 380 | 475 | 515 | 585 | 650 |
| HL | 45000 | 270 | 315 | 410 | 450 | 1045 | 1175 |
| ML | 24000 | 400 | 490 | 580 | 760 | 905 | 1030 |
| LL | 15000 | 320 | 410 | 605 | 580 | 825 | 950 |
| HO | 2000 | 125 | 220 | 410 | 490 | 630 | 760 |
| RT | GND | 35 | 125 | 320 | 400 | 540 | 670 |

## PERMISSIBLE COMMUNICATIONS ON VHF AES CHANNELS

### Regulatory References

FCC Regulations are, to the extent required for international coordination and compatibility, based upon general allocations and rules generated by the International Telecommunication Union (ITU).  FAA Rules are, in a like manner, guided by recommendations developed by the International Civil Aviation Organization (ICAO).  Both the ITU and ICAO are international bodies to which the United States is a signatory of their respective Conventions.

Use of communications channels in the AES is governed by FCC and FAA Rules, which are generally in accordance with regulations contained in the ITU Radio Regulations, and Annex 10 to the Convention on International Civil Aviation.

ICAO defines Operational Control Communications as those "required for the exercise of authority over the initiation, continuation, diversion, or termination of a flight in the interest of the safety of the aircraft and the regularity and efficiency of a flight."

The FCC defines the Aeronautical Enroute Service in similar terms:

"Aeronautical enroute stations provide operational control communications to aircraft along domestic or international air routes.  Operational control communications include the safe, efficient and economical operation of aircraft, such as fuel, weather, position reports, aircraft performance, and essential services and supplies.  Public correspondence is prohibited."

Some ATS communications are permitted on AOC channels by virtue of their relationship to safety of flight.

In addition, the FCC permits, on a secondary basis using data transmission only, Aircraft Administrative Communications, which are defined as follows:

"Such secondary administrative communications must directly relate to the business of a participating aircraft operator in providing travel and transportation services to the flying public or to the travel, transportation or scheduling activities of the aircraft operator itself.  Stations transmitting administrative communications must provide absolute priority for operational control and other safety communications by means of an automatic priority control system."

### Permissible Communications (Voice and Data)

The following are examples of typical permissible AES communications:

#### AOC

The following AOC air/ground communications are authorized on AES channels to or

from aircraft only during the period the airplane is in flight status.

- Communications relating to the initiation, continuation, diversion, or termination

of a flight

- Performance of the aircraft, including components

- Aircraft servicing

- Information of value to the crew in accomplishing a particular flight

- Information of value to ground personnel concerned with the safe and efficient

operation of a flight

- Information of value to other flights in a common geographical area

- Supplemental information pertaining to weight and balance and/or passenger

counts

- Urgent medical information

- Connections with other transportation

- Essential services and supplies

The following is a non-exhaustive list of permissible AOC communications:

    - Pilot/Dispatch Communications

      - Weather Information

      - Flight Planning Data

      - Weight/Balance

      - Flight Release

      - Flight Progress Information

      - Position Information

      - Gate Assignments

      - OOOI (Out-Off-On-In) Reports

    - Maintenance

      - Maintenance Alerting

      - Flight Diagnostics

      - Airframe/Avionics Monitoring

      - Engine Monitoring

    - Other

      - Check Lists

      - De-Icing

      - Essential Supplies and Services

    - Emergencies

      - Medical

      - Equipment

      - Non-Equipment

      - Security

      - Weather

    - Aircraft Ramp Movements

      - Base to Aircraft

      - Mobile to Aircraft

#### AAC

AAC is permissible in U.S. air space on data channels and on a secondary basis to all other permissible communications only. The following is a non-exhaustive list of permissible AAC communications:

- Travel and Transportation Services

- Non-essential Supplies and Services

#### ATS

Subject to FAA and Industry agreement, ATS communications may be accomplished on AOC channels.  For example, PDC (Pre-Departure Clearance) and ATIS (Air Terminal Information System) communications are ATS functions that are being performed on AES frequencies for the purpose of early implementation via an existing data link system for mutual FAA/Industry benefit.

### Typical Non-Permissible Communications (Voice and Data)

The following is a non-exhaustive list of non-permissible communications:

- Public correspondence

- Personal messages to or from crew or passenger

## CONTROL/DISPATCH POINTS

### Control Points

There must be a control point for each transmitter.  The control point is usually at the transmitter and must provide:

- A security system to prevent unauthorized access to and operation of the radio.

- A visual indication that the transmitter is keyed by either a dispatch point or 	the control point.

- Aural monitoring of transmissions from subordinate dispatch points.

- A way to disconnect subordinate dispatch points from the transmitter.

- A means to turn off the transmitter.

- A way to restrict calls from aircraft to the Public Switched Telephone Network (PSTN) to authorized operational control destinations.

All control points must be listed on the license. Control points cannot be located outside the U.S.  Multiple control points are permitted.

### Dispatch Points

Enroute radios can be operated from dispatch points subordinate to a control point.  Dispatch points can be located anywhere the license holder desires.  They need not be listed on the license.

Dispatch points should not be capable of operating when the associated control point is not manned.  Manned means authorized persons are within audible range of the control point.

Acceptable means of deterring unauthorized persons from using dial-up systems include:

- User programmable passwords of 4 or more characters.

- Call back schemes.

- Digital data streams.

In dial-up systems, wherein public switched circuits are used to control a ground station, the control point should not connect the telephone line to the radio until an authorized password, data stream or security tone is received.  If an authorized password, data stream or security tone is not received, the control point should automatically disconnect from the telephone line in a user programmable period of time.

Control/Dispatch points should not be accessible to unauthorized persons when not manned.

### Mobile Operations

All handheld and vehicle mounted radios are considered mobile operations.

NOTE: Handheld radios used while in a desk charger are considered base stations.

Mobile radios used on AES channels are licensed under their associated base station license and will only be used to communicate with their associated base station and/or with aircraft that are on the ground. They are not authorized to communicate with aircraft that are airborne.

## ASRI Station Inspections

ASRI employees routinely perform inspections of ASRI radio stations to ensure compliance with the FCC Code of Federal Regulations and the provisions of this manual. Radio station personnel should make every effort within reason to allow ASRI inspectors immediate access to perform inspection without prior notification. Items normally reviewed during an inspection are listed inside the front cover of the AFC Ground Station Manual. ASRI Form AS-7305 (Radio Station Inspection Report) is used as a checklist when conducting radio station inspections. An AS-7305 is completed for each licensed call sign at a particular location. Reports showing discrepancies that were not corrected on the spot are forwarded electronically to the appropriate department at ASRI Headquarters for resolution.

### Authorized Access

Upon presentation of proper identification, persons authorized access to an ASRI radio station include the following

- ASRI employees and agents on official business

- Personnel on company business

- FCC, Federal Aviation Administration (FAA), and National Transportation Safety Board (NTSB) officials or investigators engaged in official business

- Other persons as expressly authorized by ASRI Headquarters

- Technical and service personnel, such as telephone or power company technicians on official business

### Non-authorized Access

Persons not authorized access to ASRI radio station premises includes the following:

- Members of the general public

- Representatives of the press, unless specifically authorized by ASRI

- Personal visitors of radio station personnel

- Personal visitors of company personnel

- All others not specifically set forth in Section 2.6.1 covering authorized persons

## FCC Inspections and Citations

The FCC Enforcement Branch monitors and randomly inspects the operation and

oversight of ASRI radio stations. Any unsatisfactory conditions discovered during

monitoring and inspections are detailed in Citations or Notice of Violation to the station

licensee (ASRI). Citations are notices in writing from the FCC advising radio station

licensees of the existence of conditions that constitute violations of the Communications

Act of 1934 as amended, the FCC Code of Federal Regulations, or the terms of the radio

station license.

A Citation or Notice of Violation may be received at the operating location or may be

forwarded directly to ASRI Headquarters by the FCC Field Office involved. If received

at the station location, the Station POC shall contact ASRI Headquarters immediately for

disposition instructions (see inside front cover of the AFC Ground Station Manual for telephone number). In every case,

immediate action and correction of the unsatisfactory conditions are the responsibility of

ASRI. As prescribed by federal law, the FCC can and will render harsh monetary

penalties to the licensee for violations of the Communications Act, FCC rules, or terms of

the station license.

Upon receipt of a Citation or Notice of Violation, ASRI will notify the Contract Officer

of the company operating the station, providing necessary instructions for handling and

correcting any cited conditions. Corrective action must then be taken immediately after

notification from ASRI to avoid further FCC action. When corrective actions have been

completed, ASRI Headquarters must be advised with timely submission of a detailed

written report. ASRI Headquarters will make a formal reply to the FCC. At that point,

December 7, 2009 11622 Rev. H 3-9

station operations personnel may consider the matter closed unless advised otherwise by

ASRI Headquarters.

All personnel involved in dealing with FCC Citations or Notices of Violation must

remember that expeditious corrective action is mandatory and that ASRI Headquarters

must make the first reply to Citations and Notices within ten (10) days after initial receipt

from the FCC

## Interference Reporting

Interference, as used herein, refers to any man-made radio energy (as distinct from natural phenomena, such as radio energy released during electrical storms) that, when detected by a radio-receiving apparatus, is of sufficient magnitude to disturb or seriously impair reception of desired signals.  To help prevent radio station interference, installation of the equipment and antennas should be done within the guidelines of the AFC VHF Ground Station Installation Guide.  This Guide is available on the ASRI web site at www.asri.aero.

ASRI radio station operators shall record and report significant interference. All interference reports should be complete as applies to each instance and submitted immediately through the www.asri.aero web site using the on-line form.  An ASRI station receiving a report stating that signals from its station are causing interference to another station shall immediately notify ASRI Headquarters. Remedial action, within the capabilities of station personnel, shall be taken to reduce or eliminate the reported interference.  ASRI will coordinate corrective action with the

company or companies involved to promptly correct the situation.

## VHF VOICE FREQUENCY ASSIGNMENT CRITERIA

### -Channel Justification Process

An operating entity may receive assignment to a single voice channel at each location at which they operate.  Assignment to more than one frequency will be made in accordance with flight activity data. Each applicant should provide flight activity data for its own use regardless of its affiliation with other carriers. The following guidelines will apply:

The applicant will submit flight activity data to ASRI using the following formula to justify the need of additional assignment at each airport:

N = A/14

Where:

N = The number of channels required at the airport under consideration.

14 = The number of flights that can be accommodated within a channel assignment sector within a peak hour for voice.

A=i=1i=4Ti+Li+0.25PiS

Where:

Ti = the number of scheduled departures at the airport served during the four peak 15-minute activity periods.

Li = the number of scheduled arrivals at the airport served during the four peak 15 minute activity periods.

Note:  The operating entity shall select the four 15 minute periods, which need not be consecutive, but the same four 15 minute periods shall be used for all the factors, Ti, & Li.and the four 15 minute periods must be within the same 24 hour window.

Note:  Where application of the formula results in a fractional number greater than 0.3, the next larger whole number will apply.

### Congested environment procedures

What

Good voice procedures

Increased sharing requirements

### ASRI Ground Station Activity Reporting System

In its role as FCC licensee for AES frequency assignment, ASRI has an obligation to ensure suitable management of those licenses.  The ASRI Ground Station Activity Reporting System (GSARS) is an annual reporting mechanism that supports non-discriminatory use of the AES spectrum for all users, while supporting efficient use of the limited AM(R)S spectrum.

ASRI GSARS are an annual reporting requirement for all VHF licensees with more than one frequency assignment at any site.  Licensees with only one assignment at a site are not required to submit a GSAR.

A GSARS report should be filled using the ASRI website’s GSARS form page.  Annual returns can be submitted between the 1 Jan to the 31 March for the previous year.

ASRI will verify a random sample based on the data provided and site congestion, and may request supporting data to verify the calculations should inconsistencies be found.  Organizations may miss one annual GSARS report,

### Annual Frequency Utilization Reporting Requirements

The Federal Communications Commission (FCC) requires that licensees obtain the most effective and efficient frequency usage. To ensure efficient frequency utilization, ASRI implemented the Ground Station Activity Reporting System (GSARS).  GSARs is an annual reporting requirement for all ASRI voice and data frequency users to verify and mange use of the limited AES spectrum.  Such data may be requested by the FCC to verify effective usage of the AES frequency band.

#### Voice

- Voice users are required to submit a completed Channel Justification Form for each operating site/airport once a calendar year.

  - Only required for users with more than one assignment at each operating site/airport.

  - Use an online form on the ASRI website.

  - Fill in flight numbers for each 15 min period at each airport.

  - Provides result and sends it to ASRI.

- ASRI will randomly verify a small sample every year, using public data to confirm.

  - Will follow up with those users which are exceeding their justified requirement.

- Voice users who miss 2 annual reports in a row will be verified by ASRI

  - Users who do not meet requirement will be requested to provide additional data.

- Users who don’t provide the annual report after two years, or justification data, may have their assignment reassigned/shared to another user with greater need.

-

### New Users

Assignments of new operators to channels will be made in accordance with the anticipated flight traffic volume of the operation concerned.  New users will be assigned to share an existing, in-use channel unless their initial requirement is reasonably expected to meet the desired loading for a channel with a single user.  Individual users will be assigned to a non-shared channel, when available, only if that user can demonstrate a firm requirement of loading in excess of the desired loading criterion.

### Replacement assignment

Replacement of an existing assignment that is no longer acceptable for technical or operational reasons will be handled on a case-by-case basis, only after all reasonable alternative technical solutions have been considered.

### Station Facility Modifications, Additions, and Changes

The initiation of action to establish new ASRI radio stations and to change or modify the physical facilities of existing ASRI radio stations is the responsibility of the company Contract Officer.

New radio stations shall not be established and existing radio stations shall not be changed, relocated, or modified without prior authorization from ASRI, because any such changes or modifications involve the station agreement between the company and ASRI.  In many cases, this involves application to the FCC for a radio station license or modifications to an existing radio station license. ASRI shall handle all radio station licensing actions with the FCC.

Organizations staffing ASRI aeronautical enroute radio stations are not authorized to transfer, sell, assign, share, or otherwise provide use of the radio station equipment and/or the radio station frequency to any other aircraft operator, party, or entity at the ground station location without prior authorization from ASRI.

When an organization has no further requirement for service from an ASRI aeronautical enroute station that they staff, they must notify ASRI.  ASRI shall then take action to decommission the license.

### Route Adjustments

Assignments to channels due solely to route adjustments will be handled as follows:

An existing assignment provided for the users involved will be extended, insofar as possible, commensurate with maintaining interference protection in accordance with Table 2-2.  Extension of existing assignments, where required and justified, which create contention with the established operation of another user, shall be resolved by consultation and agreement among the users and ASRI.  ASRI will then initiate the required reassignment(s).

In the event an existing assignment cannot be extended without creating contention with an established operation:

- A different frequency, if available, will be selected to fulfill the new requirement

or

- The applicant involved may elect to obtain service from an established facility

or

- The air-to-air interference protection provided by the spacing shown in Table 2-2 with due consideration, may be further derogated to permit extension.

### Extraordinary Requirements

Additional assignments to channels may be considered for extraordinary or special requirements, unusual operational and/or technical problems.   Decisions regarding special circumstance will be made by ASRI management and/or the AFC.  However, any request will be required to have supporting evidence, describing in specific detail as to why such an exemption should be made.

### Network Stations

No simulcast voice network stations will be assigned to frequencies in the 136 to 137 MHz band.

The validation process for assigning voice network stations to channels will take into account the utilization rate for the entire network at the network control point.  Only "Common-user" networks will be authorized.  Assignments to network channels will be made on the same basis as "In-Range" stations, i.e. shared use.  Existing networks will be

"Grandfathered" subject to continuing compliance with the loading requirements of paragraph 2.6.3.

Network stations that also provide low altitude enroute, in-range, or ground communications service must be included in justification for additional frequencies at that location in accordance with the formula given in Section 2.6.  Those network stations which are used exclusively for high level and mid-level enroute coverage will not be included in determining  eligibility for additional channels.

### ATS Related Functions

Whenever functions typically provided on ATS channels are proposed for AES channels, additional channels from outside the AOC band should be requested due to the paucity of spectrum and the resulting reduced protection criteria used in AOC applications.

#### Ramp Control

Ramp Control assignments will be made to any certificated air carrier which has been designated by the FAA, in writing, to be responsible for the ATS ramp control function in “non-movement” areas. Frequencies required for Ramp Control in “movement” areas shall be provided by the FAA when available. Due to their shared nature, AES frequency assignments used for Ramp Control will be restricted to communications only in non-movement” areas. AES frequencies used for this purpose shall not be afforded any priority over other users’ authorized AOC communications on the same frequency. Therefore, the Ramp Control user is cautioned to assure that other means are available to communicate between the ground handlers and the flight crew in the event that the frequency is in use by others. FAA mandated ramp control assignments will not be counted in determining the number of in-range assignments of the respective user. Companies choosing to conduct “non-movement” Ramp Control functions on channels also used for other AES functions must justify the channel in accordance with the policy for AES channels.

#### De-Icing

Temporary assignments for de-icing shall not be used for any other purposes, and shall be valid only for the current de-icing season. The deicing season normally runs from the beginning of September until the end of April, but can be extended by ASRI due to unique location and/or weather situations on a case by case basis. Where de-icing requirements cannot be accommodated on currently authorized AES frequencies, , additional channels may be authorized for shared use by all users at an airport, without regard to justification by flight activity. Additional de-icing frequencies may be granted, subject to availability, up to the number of frequencies that are justified or have previously been justified at the same location in accordance with Section 2.6.1. These additional frequencies will not be reserved beyond the de-icing season, and will become available for general AOC assignments. Therefore, additional de-icing frequencies are subject to change, or lack of availability, from year to year.

## VHF DATA LINK MODE 2 FREQUENCY ASSIGNMENT CRITERIA

Common-user data link systems based on industry derived standards, have demonstrated increased spectral and economic efficiencies in providing integrated data link communications networks and are supported by the aviation community as a spectrally efficient means of providing air-ground data communications.  This policy addresses common-user ACARS and VDLM2 systems, which allow networked multiple ground stations to use the same frequency within overlapping service volumes. Therefore, only common-user data link systems will be authorized. All applications for new data link systems must be approved by the AFC.

### 2.7.1	General

Due to the extreme shortage of AES frequencies, it may not be possible to grant authorization for data link frequencies justified in accordance with this section. When a suitable data link frequency is not otherwise available, it may be possible to relocate incumbent users from a frequency identified by ASRI Frequency Management. It shall be the responsibility of the applicant to negotiate the relocation of the incumbents.

An additional frequency is justified only when the peak RF channel activity regularly exceeds a given occupancy on all currently assigned channels simultaneously, as defined and/or measured in accordance with Section 2.7.2.  (This percentage reflects today's technology and should not be construed to be an impediment to future optimization.)  The ground station operator will determine channel loading by physical measurement of RF channel occupancy, or by analysis of traffic records, in accordance with Section 2.7.2 .

An additional frequency may be assigned if a trend can be shown that would cause all currently authorized system frequencies to be loaded in excess of the metrics defined in Section 2.7.2 within one year. Frequencies authorized under this provision will be re-evaluated after one year, and will be decommissioned if not justifiable at that time.

All under-utilized frequencies at a particular station are subject to return to the common pool of assignable frequencies. A single under-utilized frequency will not be subject to recall. However, additional under-utilized frequencies will be returned to the pool after one year.

A frequency that is under-utilized at a particular station will not be subject to return to the common pool if the exclusion zones of remaining justified co-channel stations preclude its use for other purposes.

Data operations shall conform to the protocols and other signal-in-space standards set forth in ARINC Characteristics 597A, 724, 724A, 724B, 750, 758, ARINC Specifications 618, 619, 620, 622, 623, and 631, and their successors, to assure proper discipline and efficiency in the use of the spectrum.

All data ground stations shall have the ability to deliver third party data traffic if requested and prior arrangements have been concluded.

Systems transmitting administrative communications shall employ an automatic priority system to ensure absolute priority of Safety (ATS and AOC) communications on the channel whether these communications originate in the aircraft system or in the ground system.

### 2.7.2	Methods of Data Link Frequency Justification

Channel loading shall be determined as "seen" by an aircraft flying at the maximum coordinated altitude above the location at which an additional frequency is proposed ("test point"). The VDLM2VDLM2 system has a more efficient channel access algorithm than the ACARS system resulting in a much improved channel performance (loading) per channel.

#### 2.7.2.1 ACARS

##### 2.7.2.1.1 RF Monitoring Method

An additional frequency will be considered as justified when RF monitoring at or below the test point shows that all currently authorized frequencies are simultaneously loaded at, or above, 40% over the averaging period. The averaging period is defined as one hour. Alternatively, if more than one frequency is assigned; three five-minute periods in one month may be used.

##### 2.7.2.1.2 Traffic Analysis Method

An additional frequency will be considered as justified when an analysis of the traffic records shows that all currently authorized frequencies at that location are simultaneously loaded at, or above, 40% over an averaging period of one hour. This analysis will be based upon the following parameters:

It may be assumed that 2000 ACARS block attempts per hour is equivalent to an RF channel load of 40%. System control blocks with labels listed in ARINC Specification 620 Section 4.2 (Uplinks) and Section 5.2 (Downlinks) may not be counted for this purpose.

All uplinks from ground stations within line-of-sight of the test point may be considered.

All unique downlinks received from stations within 1.66 times line-of-sight of the test point may be considered.

The total uplinks and downlinks recorded for each system frequency currently authorized, and for the same time period, will be tabulated.

For additional information on the derivation of these requirements, see AFC FP 99-07.

#### 2.7.2.2 VDLM2

##### 2.7.2.2.1 RF Monitoring Method

An additional frequency will be considered as justified when RF monitoring at or below the test point shows that all currently authorized frequencies are simultaneously loaded at, or above, 40%  during the busy hour over the averaging period. The averaging period is defined as over a 30-day period, the number of days which the 40% threshold is exceeded is greater than 15 days.

##### 2.7.2.2.2 Message Latency Method

The message latency measurement metric is the time between the 620 message arriving at the CSP Service Delivery Point (SDP) and successful reception of the corresponding 618 message at the aircraft antenna for uplink messages and the time between queuing the 618 message at the aircraft and delivery of the corresponding 620 message to the CSP SDP for downlink messages.

- For each airport or en route service volume:

  - Linearly extrapolate the monthly 95% latency values out 18 months

    - Based on the last 12 months

    - Based on the last 6 months

  - If either projection approaches 9 seconds at 18 months:

    - Verify growth in data throughput

    - Verify that the increase in channel utilization is commensurate with data traffic growth (i.e. not due to other issues such as hardware failures or RF interference)

- Other factors that may be considered:

  - Pending changes in customer base, fleet composition or customer VDLM2 CON OPS that indicate an imminent increase in data loading

  - Analysis and predictions indicating latency issues – for example, the initiation of Data Com en route operations in 2019

If the projection is approaching 9 seconds (> 7 seconds) at 18 months and the data throughput and channel utilization support the conclusion that the latency is due to data traffic, then an additional frequency channel can be requested and should be assigned.

## 2.8	U.S.A./CANADA INTERIM CHANNELLING ARRANGEMENT FOR THE AERONAUTICAL MOBILE (R)/(ENROUTE) SERVICE UTILIZING 25 KHZ CHANNELS FOR THE BAND 128.8125-132.0125 MHZ

This Arrangement, which is subject to periodic review at the request of either Administration, is of an interim nature pending its incorporation into an Allotment Plan for certain bands allotted to the aviation services to meet the United States and Canadian aeronautical frequency requirements along the U.S./Canada border, to the mutual satisfaction of both countries.

Implementation of 25 kHz channel assignments by either Administration shall become effective on the date of signing of Letters of Understanding by both the United States and Canada.  The use of 50 and 100 kHz equipments shall not be protected from properly operating 25 kHz equipments beyond February 1, 1981.

The provisions of this Arrangement apply to Canada and the United States in the utilization of the 25 kHz channels for the band 128.8125-132.0125 MHz which are shown in Table 2-3.

NOTE:	The future implementation of 8.33 kHz channel spacing into the band 128.8125 to 132.0125 MHz will require re-evaluation of the U.S.A./Canada Interim Channeling for the Aeronautical Mobile (R)/(Enroute) Service Agreement.

Table 2-3

Freq. MHz U.S.A. Canada Freq. MHz U.S.A Canada

128.825 X 129.625 X

128.875 X 129.675 X

128.925 X 129.725 X

128.975 X 129.775 X

129.025 X 129.825 X

129.075 X 129.875 X

129.125 X 129.925 X

129.175 X 129.975 X

129.225 X 130.025 X

129.275 X 130.075 X

129.325 X 130.125 X

129.375 X 130.175 X

129.425 X 130.225 X

129.475 X 130.275 X

129.525 X 130.325 X

129.575 X 130.375 X

130.425 X 131.225 X

130.475 X 131.275 X

130.525 X 131.325 X

130.575 X 131.375 X

Table 2-3 (cont’d)

Freq. MHz	U.S.A.		Canada		Freq. MHz	U.S.A		Canada

130.625	   X				131.425	   X

130.675			    X		131.475			    X

130.725	   X				131.525	   X

130.775			    X		131.575			    X

130.825	   X				131.625	   X

130.875			    X		131.675			    X

130.925	   X				131.725	   X

130.975			    X		131.775			    X

131.025	   X				131.825	   X

131.075			    X		131.875			    X

131.125	   X				131.925	   X

131.175			    X		131.975			    X

Table 2-3 (cont.d)

Freq. MHz		U.S.A.		Canada

136.525	  	  X

136.575				   X

136.625		  X

136.675				   X

136.725		  X

136.775				   X

136.825		  X

136.875				   X

136.925		  X		   X

136.975		  X		   X

Note 1)	Frequencies primarily allotted for United States use may also be assigned to Canadian stations within the frequency coordination zones shown in Figure 2-6, following successful coordination with the United States.

Note 2)	Frequencies primarily allotted for Canadian use may also be assigned to United States stations within the frequency coordination zones shown in Figure 2-4 and Figure 2-5, following successful coordination with Canada.

Note 3)	All frequencies listed in Table 2-3 may be assigned without prior coordination outside the indicated coordination.

Channels proposed for assignment in accordance with Table 2-3, which will be located within 25 NM of the U.S./Canada border, shall be coordinated with the other Administration prior to implementation.  Requests for channel assignments removed 25 kHz from existing assignments listed in Table 2-4 will be coordinated when such an assignment will be within 130 NM of the existing station.  With these exceptions, channels may be assigned and stations established by either country in accordance with Table 2-3 without coordination with the other country.

Table 2-3 (cont.)

In adopting the above arrangement, it is understood that each administration has complete flexibility in making use of its channels within the frequency coordination zone; that the option provided by Notes (1) and (2) should not be exercised unless the proposed assignment cannot be accommodated on a channel allotted under the plan; that an assignment made under the provisions of Notes (1) and (2) should not be a bar to future utilization of the channel by the Administration to which it is allotted under this arrangement.  However, in the event that return of a frequency is deemed necessary by the Administration to which it is allotted, consideration shall be given to the overall frequency utilization in the area of the proposed station.  Before an Administration requests the return of one of its allotted frequencies, it shall be established that it cannot otherwise meet its operational requirements.  Where a determination indicates congestion of equal magnitude, consideration should be given to the sharing of a frequency on an equal basis by the two countries.  In cases where such determination indicates greater congestion in the area of the proposed station, the frequency shall be returned to the Administration to which the channel is allotted.

In order to ensure protection of the Air Traffic Control frequencies 128.8 and 132.025 MHz, the assignment of the frequencies 128.825 and 131.975 MHz within 600 NM of the U.S./Canada border are subject to prior coordination with the other Administration.

Table 2-4

U.S. 25 kHz Assignments will be

coordinated with Canada when

adjacent to and within 130 Nautical Miles of

the existing 50 kHz Canadian

assignments listed hereunder

| LOCATION | FREQUENCY (MHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Abbotsford, British Columbia | 129.25 | 130.80 |  |  |  |  |
| Burnaby, British Columbia | 130.00 |  |  |  |  |  |
| Delta, British Columbia | 128.85 | 130.80 |  |  |  |  |
| Saltspring Island, British Columbia | 130.50 |  |  |  |  |  |
| Vancouver, British Columbia | 129.25 | 129.35 | 129.90 | 130.80 | 130.90 | 131.20 |
| Victoria, British Columbia | 129.20 | 130.60 |  |  |  |  |
| Fort Frances, Ontario | 130.15 |  |  |  |  |  |
| Sarnia, Ontario | 130.35 |  |  |  |  |  |
| Sault Ste. Marie, Ontario | 130.15 | 130.90 |  |  |  |  |
| St. Catherines, Ontario | 131.10 |  |  |  |  |  |
| Windsor, Ontario | 131.10 |  |  |  |  |  |

Table 2-5

FREQUENCY COORDINATION ZONES

FOR CO-CHANNEL ASSIGNMENTS

PER U. S./ CANADIAN AGREEMENT2121

Type Of Station		Altitude Level2222		 Coordination Zone

(Feet)			  (Nautical Miles)

Ramp (RT)			Ground Level				50

Helicopter (HO)		0 to 2,000				150

Low Level (LL)		0 to 10,000				250

Mid-Level (ML)		0 to 20,000				400

High Level (HL)		Over 20,000				600

Note that due to the "sterile cockpit rule" coordination altitudes for AES stations in the U.S. are different from those in the Canadian agreement.  See Tables 2-1 and 2-2 for AES coordination levels.  Thus, a U.S. Low-Level assignment should be coordinated with Canada as a Mid-Level assignment.

## 2.9 	UTILIZATION OF THE AES FREQUENCY BAND 136.500 - 137.000 MHz

2.9.1 Voice Mode Spectral Plan using 8.33 kHz Channel Spacing

To support initial 8.33 kHz implementation it was recommended to utilize a sub-band of 100 kHz starting at frequency 136.500 MHz and ending at frequency 136.600 MHz.  This sub-band will be used for systems employing voice communications modes only when all lower frequencies have been fully assigned or for digitally modulated data communications systems only when all higher frequencies have been fully assigned. (Use of frequencies in this sub-band in the U.S. or in Canada must be coordinated with Canada or the U.S. in accordance with the U.S. Department of State “Treaties and Other International Acts Series 5205”).

2.9.2 VDLM2 Spectral Plan

2.9.2.1 Frequency Separation and Installation Requirements

VDLM2 communications use a D8PSK modulation operating at a data rate of 31.5 Kbps.  It is considerably less tolerant of interference as compared to classic ACARS operation, which uses Amplitude Modulation (AM) and a sub modulation of minimum shift key (MSK) at a data rate of 2400 bps.  As a result, special operational and field considerations are required in order to ensure successful operation of the VDLM2 service.   The following material was developed based on extensive analysis and testing and provides the fundamentals of a successful VDL spectral plan.

2.9.2.1.1 Co-Site Operation of Active VDL Channels

For co-site installations where the VDL antenna to VDL antenna separation must be a minimum of 60 feet horizontal and at least 150 kHz of separation or five 25 kHz guard-band channels are required for a standard VDL transmitter power of 25 watts.

In addition, to prevent interference, tuned-cavity notch filters are required for each VDL frequency that is operated in this co-site environment.  It is expected that the VDL service will require three active VDL channels in an airport, co-site environment (e.g., the Common Signaling Channel (CSC), Alternate #1, and Alternate #2).  Therefore, it is expected that each VDL transceiver will require two cavity filters with the notch tuned to the other two frequencies that are being rejected.

A VDL transceiver operating on the CSC will require two serially connected filters between the transceivers and antenna, one notch tuned to Alt #1 and one notch tuned to Alt #2 is shown in Figure 2-7.

Figure 2-7

2.9.2.1.2 Required Performance of TX/RX Cavity Filters

Performance of cavity filters can vary considerably by both manufacturer and model.  While use of cavity filters can reject off-tune signals, cavity filters have the adverse effect of adding insertion loss to the signal path.

Suitable cavity filters for this application should yield a minimum rejection of 20 dB at a 125 KHz offset and a pass frequency insertion loss of ideally 1.0 dB or less.

Data below is provided for two suitable filters: a 6 inch Vari-Notch filter and a 10 inch Vari-Notch filter.   The 10 inch filter is physically larger and more costly.    However, with increased notch rejection and reduced insertion loss, a 10 inch Vari-Notch filter is recommended. Cavity filter insertion loss and rejection levels are illustrated in Table 2-6.

Table 2-6

| Frequency separation | TX/RX 6” Vari-Notch |  | TX/RX 10” Vari-Notch |  |
| --- | --- | --- | --- | --- |
|  | Insertion loss (dB) | Rejection (dB) | Insertion loss (dB) | Rejection (dB) |
| 75 kHz | 1.1 | 14.5 | 1.7 | 20 |
| 100 kHz | 1.5 | 20 | 1.0 | 21 |
| 125 kHz | 1 | 20 | 0.8 | 24 |
| 150 kHz | 1 | 23 | 0.9 | 27 |
| 175 kHz | 0.7 | 24 | 0.5 | 27 |

One family of cavity filters is the Vari-Notch Cavity filters made by Bird Technologies Group.  These filters must be ordered as a pass low reject high filter or a pass high reject low filter depending on the deployment configuration.  The tuning of these filters may need to be adjusted on-site because of mechanical vibration and shock during shipment.

2.9.2.1.3 Mixed Mode Operations

VDLM2 operations can co-exist with other ACARS (AM with MSK modulation), VDLM2 data, and voice services if the proper frequency separation is observed.  Based on a transmitter power of 25 watts and omni-directional antennas:

- A VDLM2 station can be successfully operated within 50 kHz of another VDLM2, AM Voice, or ACARS station if the antennas are separated by a minimum of 1.0 mile.

- Some interference (~10-12% channel degradation) can be expected if the stations are operated with separation of less than 1.0 mile.

- Unacceptable interference and degradation can be expected if the stations are operated closer than 0.5 mile unless proper filtering is applied.

2.9.2.2 VDLM2 Implementations

The implementation considerations include frequency assignments based on philosophy of deployment.  Variables include shared vs. dedicated channels, potential AOC and ATC frequency sharing, transmitter power output, transmitter intermodulation products, and the relationship between frequency and physical separation.  These topics are discussed in this section.

2.9.2.2.1 Shared Common Signaling Channel and Dedicated Channels

Industry standards and agencies have identified 136.975 MHz as the international common signaling channel (CSC) for VDLM2 operations.  All data link service providers announce their service availability on this channel.

Per VDLM2 industry standards, to avoid congestion and off load traffic, each data link service provider may operate one or more VDLM2 alternate channels.  Unlike the CSC, these channels are not shared but are dedicated and licensed to each DSP.   Dedicated channels avoid the issues associated with shared channels (CSMA collisions, hidden terminal effects, and interference) and maximize the efficient use of the VHF spectrum.

2.9.2.2.2 Coordination of ASRI AFC and FAA VDL Spectrum for AOC and ATS Applications

From the upper portion of the Aeronautical Enroute Service (AES) 136-137 band, the FAA shares the 136.000 to 136.475 MHz portion of the band with the FCC.  This is used mostly for FAA ATC applications and other federal users .  ASRI has been allocated the 136.500 to 136.975 MHz portion of the band for AES applications.

The current direction as indicated by FAA documentation is that the FAA data link based Data Communications program will use VDL channels that share both AOC and ATS applications.

Although a formal plan is unavailable it is expected that, when required for the Data Communications program, the FAA will allocate frequencies than can be used for ATC and AOC applications.  This plan will be defined in the future.

2.9.2.2.3 Airport Installations with Multiple VDL Transceivers

Due to the complex nature of larger airports and based on airlines requirements for redundancy and comprehensive VDL coverage at all gates, data link service providers frequently operate several ground stations that share the same frequencies at the same airport.

In airports where multiple VDLM2 Transceivers are used, the service provider may designate a station as the primary “enroute” station operated at full transmitter power, typically 25 Watts.  In order to minimize unnecessary spectral emissions, the other VDL transceivers within the airport shall operate with the minimum power needed to provide successful communications at the gate areas.

2.9.2.2.4 Transmitter Intermodulation Considerations

When two collocated transmitters are activated, third order intermodulation products are produced.  Even with the cavity filters installed, these products are about 10dB stronger than the typical adjacent frequency emissions of the transmitters.

The transmitter intermodulation products will be generated at frequencies 2F1 – F2 and 2F2 – F1.  If there is no channel assigned at these offset frequencies, then the intermodulation products are transmitted without detriment to any of the VDL channels.  As an example, if we have 136.975 MHz and 136.825 MHz (150 kHz offset), the third frequency cannot be 136.675 MHz (another 150 kHz offset) because the 2F1-F2 product is on the CSC 136.975 MHz.  Because of this, the first DSP is assigned to 136.800 MHz or 175 kHz from the CSC and the second DSP is assigned a frequency that is 150 kHz away or at 136.650 MHz

2.9.2.2.5 Frequency Separation vs. Geographical Separation

Transmitter to receiver isolation may be achieved by a combination of frequency and geographical separation.  The tables below are based on the adjacent channel emissions as specified in section 6.2.4 of the ICAO SARPS.  Note the inverse relationship between antenna separation and frequency separation.

The SARPS specifies that the adjacent channel emission for the fourth adjacent channel is at a maximum of -38 dBm and that it shall monotonically decrease at a rate of 5 dB per octave thereon.  The free space loss will increase by 6 dB each time the geographical isolation doubles.  These two points are the basis for the VDLM2 Frequency Separation versus Antenna Separation table below. VDLM2 frequency separation versus antenna separation with cavity filters is shown in Table 2-7.

Table 2-7

| Frequency Separation | Emissions at | Notch Filter Isolation | Minimum Antenna | Minimum Antenna |
| --- | --- | --- | --- | --- |
| kHz | this Separation | 10 Inch Cavity Filter | Separation | Separation |
|  | dBm | dB | feet | Miles |
| 25 | 2 | 0 | 169103 | 32.027 |
| 50 | -28 | 0 | 5348 | 1.013 |
| 75* | -33 | 20 | 301 | 0.057 |
| 100* | -38 | 21 | 151 | 0.029 |
| 125* | -39 | 24 | 95 | 0.018 |
| 150 | -40 | 27 | 60 | 0.011 |
| 200 | -43 | 27 | 42 | 0.008 |
| 400 | -48 | 27 | 24 | 0.005 |
| 800 | -53 | 27 | 13* | 0.003 |

*Note 1: This table does not consider the undesirable insertion losses associated with the operation of a notch filter that is tuned to reject interference separated by 75 to 125 KHz from the desired (Pass) frequency.

Note 2: Placing radios closer than 18 feet together without cavity filters may have destructive effects on the front end of the receiving radio.

Table 2-8

| Frequency separation | Emissions at | Minimum Antenna | Minimum Antenna |
| --- | --- | --- | --- |
| kHz | this separation | Separation | Separation |
|  | dBm | feet | Miles |
| 25 | 2 | 169103 | 32.027 |
| 50 | -28 | 5348 | 1.013 |
| 75 | -33 | 3007 | 0.570 |
| 100 | -38 | 1691 | 0.320 |
| 125 | -39 | 1507 | 0.285 |
| 150 | -41 | 1197 | 0.227 |
| 200 | -43 | 951 | 0.180 |
| 400 | -48 | 535 | 0.101 |
| 800 | -53 | 301 | 0.057 |

Table 2-8 illustrates frequency separation versus geographical separation with no cavity filters. It is expected that the emissions of the radio will continue to decrease as the frequency separation increases and that the minimum antenna separation will continue to decrease.  The table does not go beyond the SARPS requirement that the emissions continue to decrease to a maximum of       -53dBm (second column).

2.9.3 Proposed Voice and VDL Frequency Assignments

The proposed frequency plan for the upper portion of the Aeronautical Enroute Service sub-band from 136.500 to 136.975 MHz is provided in Table 2-9.

To support initial 8.33 kHz implementation it was recommended to utilize a sub-band of 100 kHz starting at frequency 136.500 MHz and ending at frequency 136.600 MHz.

This plan identifies two VDLM2 On-Airport alternate frequencies which may be operated in a co-site environment (60 feet of horizontal separation).  It is expected that 3 frequencies, the CSC, DSP Alternate #1 (136.650 MHz), and DSP Alternate #2 (136.800 MHz) will provide sufficient AOC capacity for the next decade.   Thereafter enroute frequencies can be assigned as needed based on the description below.   Enroute assignments require a minimum of 1 mile of separation between the transmitter antennas providing for the CSC and two VDL On-Airport alternates.  The VDL enroute frequencies could alternately be assigned for Voice or ACARS operations given the separation requirements of >1.0 mile are met.

This plan only considers the ASRI upper part of the VDL spectrum and does not consider the 128 discrete 25 kHz channels in the lower Aeronautical Enroute Service sub-band of 128.825 to 132.000 MHz.  The FAA is expected to contribute some VDLM2 channel for the expansion of ATS applications.

Table 2-9

| VHF Frequency (MHz) | VDL or Voice (8.33 kHz) Allocation | Notes |
| --- | --- | --- |
| 136.975 | Common Signaling Channel | VDLM2 |
| 136.950 | Guard Band Channel | Unused |
| 136.925 | VDL Enroute Alternate (>1mi) | Voice/VDLM2/ACARS |
| 136.900 | Guard Band Channel | Unused |
| 136.875 | VDL Enroute Alternate (>1mi) | Voice/VDLM2/ACARS |
| 136.850 | Guard Band Channel | Current SITA USA ACARS Base |
| 136.825 | Guard Band Channel | Unused |
| 136.800 | VDL Alternate #1 On Airport | Co-Site (60 feet) |
| 136.775 | Guard Band Channel | Unused |
| 136.750 | VDL Enroute Alternate (>1mi) | Voice/VDLM2/ACARS |
| 136.725 | Guard Band Channel | Unused |
| 136.700 | VDL Enroute Alternate (>1mi) | Voice/VDLM2/ACARS |
| 136.675 | Guard Band Channel | Unused |
| 136.650 | VDL Alternate #2 On Airport | Co-Site (60 feet) |
| 136.625 | Guard Band Channel | Unused |
| 136.600 | VDL Enroute Alternate (>1mi) | Voice/VDLM2/ACARS |
| 136.575/136.5833/136.5917 | Voice (8.33 or 25 kHz) or VDL Guard Band Channel | Voice/Unused |
| 136.550/136.5583/136.5667 | Voice (8.33 or 25 kHz) or VDL Enroute Alternate (>1mi) | Voice/VDLM2/ACARS |
| 136.525/136.5283/136.5417 | Voice (8.33 or 25 kHz) or VDL Guard Band Channel | Voice/Unused |
| 136.500/136.5083/136.5167 | Voice (8.33 or 25 kHz) or VDL Enroute Alternate (>1mi) | Voice/VDLM2/ACARS |

FIGURE 2-4

FIGURE 2-5

FIGURE 2-6

# VHF Installation Standards

## 3.1	Introduction

These guidelines provide direction and recommendations for the installation of aviation VHF Ground Stations that are licensed by ASRI in the AES bands.

Generally, the most recent licensee deployed at a location is responsible for mitigating any and all interference caused by the addition of their system.  Unless the existing licensee is not in compliance with the terms of their license.  Then the responsibility may be shared between the involved licensees, or even entirely the existing licensee if deemed necessary by ASRI.

Mitigation measures may include, but are not limited to, the purchase and installation of cavity filters on other tenant’s systems. Stated differently, it is understood that any new installation that causes harmful RF interference to an existing user Base Station will be resolved at the expense of the new licensee.  The standards herein have been developed in an attempt to minimize such situations.

## 3.2	Frequency Management (pre-installation/site survey)

- Identify all existing co-located base stations.  A co-located base station is defined as any base station that operates on an antenna within 200’ of the proposed antenna location.

- Determine/measure the distance between all co-located antennas and the location of the proposed antenna and record the results.  It is highly recommended that maximum antenna separation be achieved between base stations.  This provides for greater transmitter isolation with co-located base stations, minimizing the risk of transmitter intermodulation.

- Identify the operating frequency of these co-located stations and correlate them to the antenna distances above.

- Recommend an Inter Modulation study be done on all frequencies located within 200 feet of the station being installed.  The study should identify all 3rd and 5th order products that fall within the Aeronautical Frequency band.  Consult with ASRI to determine if these frequencies are in use at this location. If they are, coordination with the using agencies should be done.

## 3.3	Antennas

- The antenna should be installed using a vendor approved mount only.

- The antenna mount and ancillary hardware should be either stainless steel or hot-dipped galvanized steel only.

- The connection between the antenna and the transmission line (and all other RF connections external to the building) must be weather proofed.

- All decommissioned and unused antennas should be removed from the site.

- Typically base station antennas are assumed to be omni-directional with unity gain (2.15 dBi)

## 3.4	Transmission lines

- All outdoor transmission lines shall be of the solid outer conductor type. Good quality cable should be used. Examples include the Heliax or LMR series type.

- Transmission lines that are in excess of 60’ in length external to the building or shelter must be grounded within 2’ of the antenna connection using a vendor approved grounding kit and connected to the building grounding system.  The grounding kit must be weather proofed after installation using butyl rubber or vulcanizing tape; either of which should then be encapsulated with electrical tape.

- All transmission lines must be grounded within the building or shelter using a vendor approved grounding kit bonded to earth ground.

- All transmission lines must be terminated with a vendor approved connector.  It is highly recommended that connectors using ferrous metals (nickel) not be used due to corrosion and them being a possible source of Passive Intermodulation.

- The indoor connector should terminate at an approved surge suppression device (Polyphaser).

- All transmission lines should be labeled at the following locations:  at the antenna, at the building penetration and at the radio (or cavity).  The labels shall indicate the owner and the transmission frequency. All labels shall be installed on the transmission line in a manner/location that are visible from common areas (one should not be required to open cabinets or racks to find the labels).

- All “spare” transmission lines that are installed for future use/maintenance (or decommissioned) should be properly terminated with a 50 ohm load on at least one end and properly weatherproofed and marked.

- All transmission lines used between indoor equipment (commonly referred to as jumpers) should be of the double shielded coax type (RG-223, RG-393, RG-142,  etc.).

## 3.5	Rack/Equipment

- All equipment must be FCC type approved.

- All radios should be installed in secure cabinets or be in secure areas.

- Radios should be in areas protected from excessive heat and moisture.

- All equipment racks are to be properly grounded to the building earth ground system.

- All equipment that has provisions for an external earth ground should be connected to the building system ground.

- All equipment should be secured in the rack using the vendor recommended mounts and fasteners.

- All transmitters must be labeled with their operating frequency on their face.

## 3.6	TELCO

- For installations in remote locations not serviced directly by an airport telecommunications room, it is recommended that surge protection be installed on all telco lines.

## 3.7	RF Isolation (transmitter IM prevention)

In general, to minimize the possible occurrence of IM interference, each system should obtain a nominal RF isolation of 45 dB from all other transmitters. This is equal to 60' horizontal separation between antennas in the aviation VHF band. This is achieved with 25’ of vertical separation between antennas in the aviation VHF band. If vertical separation is not an option, then 60’ of horizontal separation will provide approximately 30 dB of isolation.

The following tactics should be used for installations in a moderate to high RF location, when additional isolation is required.

- Maximize antenna separation between co-located base stations and determine the free-space loss between each existing antenna and the proposed antenna location.

- Using the TnRd curves for the specified radios, determine what the required isolation values are for both the transmitter noise and transmitter carrier.

- Some guidelines are provided below based on the Motorola VHF Quantar 100 Watt VHF station transmitter sideband noise specifications and adjacent channel rejection ratio for the receiver curves

  - A transmitter to receiver separation of 0.2MHz to 1 MHz requires 73dB + 6dB margin = 79dB transmitter noise filtering to not have transmitter noise above the –123dBm Noise floor of the receiver.  At 2MHz, 76dB is required, at 3MHz, 75dB is required and at 5 MHz, 71dB is required.

  - A transmitter to receiver separation of 0.2MHz to 2 MHz requires 66dB + 6dB margin = 72dB transmitter carrier filtering to prevent a single 100 Watt transmitter from degrading the receiver sensitivity.  At 5MHz separation, 69dB is required.

- If the transmitters have a single stage isolator built into them, the use of an external isolator on the transmitter multicoupler is not required when there is at least 45dB filter isolation between transmitters on the same antenna.

- Choose appropriate equipment to meet the required isolation values.  For example, the use of C2037 as transmitter combiners at 3.2dB insertion loss will provide 15dB isolation at 150 KHz and 45dB at 300 KHz.  At 1.7 dB insertion loss, these values are achieved at 250 and 650 KHz respectively.  When set to 3.2dB insertion loss, the receiver frequencies can be as close a 1.3 MHz away on the same antenna or 0.4 MHz on separated antennas to achieve 79dB isolation.

- Increase the transmitter conversion efficiency (IM attenuation). The only effective way to achieve this is to consider replacing the transmitter with a transmitter that has better performance characteristics.  Therefore, this should be considered a last resort.

## 3.8	Interference Identification, and Elimination

As the proliferation of VHF air to ground base radios continues, the incidence of radio interference (RFI) is increasing. Identifying the source of the interference often can be very difficult. Sometimes the source of the interference is the sufferer's own equipment.

## 3.9	Technical Terms and Their Meanings Related to Interference

### 3.9.1	 Intermodulation

Intermodulation (IM) or intermodulation distortion (IMD) is a frequency conversion

process that occurs when two or more signals pass through a non-linear system or device(s)/component(s) within a system. The essential result of the process is that energy contained in the input signal of a non-linear system is transformed at its output into a set of frequency components at the original frequencies plus additional components at new frequencies that were not contained in the input. The IM phenomenon is often referred to as mixing.

For example, consider a signal composed of two fundamental tones f1 and f2 that could represent two transmitter signals co-located at a communications site. If this composite signal is passed through a non-linear device (of third-order), the most general form of the output signal will contain frequency components at dc, f1, f2, second-order products and harmonics as well as the third-order products at 2f1 - f2, 2f2 - f1. These last products are often troublesome because they fall closest to the original tones at f1 and f2. It is possible that the newly generated third-order products could fall close to or within the receive band of a communication system located at the same site, which could degrade the performance of the receiver.

As another example, consider the same two tones at f1 and f2 passing through a stronger non-linear device of fifth-order. The set of most potentially troublesome IM products that can be produced by a fifth-order non-linear system would fall at the frequencies

2f1 - f2, 2f2 - f1, third-order products,

3f1 - 2f2, 3f2 - 2f1, fifth-order products.

Note that the order of the non-linearity is determined by the sum of the coefficients. If the non-linearity were stronger still (such as a seventh-order), it would have an output containing the following most potential interfering carriers

2f1 - f2, 2f2 - f1, third-order products,

3f1 - 2f2, 3f2 - 2f1, fifth-order products,

4f1 - 3f2, 4f2 - 3f1, seventh-order products.

With respect to the original tones at f1 and f2, the third-order components are closest, the fifth-order are the next closest and the seventh-order are furthest removed but still ‘close’ to f1 and f2. This pattern continues for devices of increasing non-linear severity.

When more than two tones of sufficient strength are present at a site, the generated IM products will consist of the set of tones occurring at all linear combinations of the original tones (up to the order of the non-linearity). Some of these IM tones will be potentially threatening to system performance, with the exact threat being dependent upon the particular frequencies and bandwidths of the receivers present at the site.

Figure 3-1: Intermodulation Products

The third order-difference intermodulation products generated by two sources are usually the most serious due to the fact that they fall within the same aeronautical band: 2F1-F2 and 2F2-F1. Fifth order products 3F1-2F2 and 3F2-2F1 also fall within the same band but are normally much less amplitude. For example if F1 equaled 130.4 MHz and F2 equaled 130.6 MHz the following products could be generated:

130.2 MHz, 130.8 MHz (3rd order)

130.0 MHz, 131.0 MHz (5th order)

Note: For transmitters, the European Telecommunications Standards Institute (ETSI) EN 300 676 Standard, section 7.8, defines a test method and specifies limits for intermodulation attenuation caused by the presence of the carrier and an interference signal entering the transmitter via the antenna. This test method is a good way to determine any deficiencies in transmitter design. Any modern equipment should be able to exceed the specified limit. See Section 2.3 "Calculating Transmitter Intermodulation Susceptibility".

### 3.9.2 Blocking or Desensitization

Blocking or desensitization results when a strong unwanted signal at the receiver input causes a change in the desired signal level.  This unwanted signal can effectively “block” the desired signal, thus the term.  ETSI EN 300 676 section 8.9 defines a test method and specifies limits for the blocking ratio.  The limit specified for a base station, shall not be less than 80 dB.  Modern VHF base station receivers should be 10 to 20 dB’s above the ETSI limit.

### 3.9.3 Spurious Emissions

Spurious emissions are any emissions from a transmitter which are not part of the theoretical output.  Any radiation from a receiver (normally local oscillator leakage) is spurious.  Section 87.139 of the FCC rules specifies the limits associated with this measurement for type acceptance (TA).  ETSI EN 300 676 section 7.5, 7.6 and 7.7 defines a test method and specifies limits for these emissions but also includes near in noise and adjacent channel power limits.  ETSI EN 300 676 section 8.10 defines spurious radiation related to the receiver.

### 3.9.4	Cross Modulation

Cross modulation in regard to receiver operation is related to the transfer of modulation to the desired signal from a strong adjacent transmitter.  Unlike “blocking or desensitization” where the undesired signal attenuates the desired signal cross modulation appears along with the desired signal.  Common cases of this are due to receiver front end design and “local oscillator” noise allowing the mix to take place.  This mix can be from the synthesizer noise floor of either the transmitter or receiver.

## 3.10	Reducing Interference Related to Airport Installations

Interference is normally related to intermodulation, blocking or desensitization, spurious emissions and/or cross modulation due to transmitters too closely coupled to receivers.  To resolve this we need to add attenuation between those transmitters or receivers by increasing separation distance or by additional filtering.  Additionally, the design of the equipment plays a major role as to how well the equipment will play together.

A quick way to determine if the intermodulation is being generated within the receiver front end is to add a small attenuator in the path.  If the interference drops by other than the amount of the attenuator, the interference is caused by the receiver.  Example a 3 dB attenuator caused a 9 dB reduction of the interference level.  This also applies when using a Spectrum Analyzer to determine if the test is valid.   If the interference drops by the amount of the attenuator then the problem is external (most likely coupling between two other transmitters).

### 3.10.1 Space or Path Attenuation

The approximate straight line path attenuation space to radio waves is given by the formula:

L = 36.58 + 20 Log F + 20 Log D

Where

L = path loss in decibels,

F = Frequency in MHz, and

D = distance between points in statute miles.

Using the above formula at aeronautical VHF:

60 feet is equal to 40 dB path loss

(This relationship is important to remember)

200 miles is equal to 125 dB path loss

(For a 20 watt ground transmitter an aircraft receiver 200 miles away would have a signal level -82 dBm)

If we double the distance we increase the path loss by 6 dB. If we halve the distance we decrease the path loss by 6 dB.  The following table might make this clear:

15 feet is equal to 28 dB path loss

30 feet is equal to 34 dB path loss

60 feet is equal to 40 dB path loss

120 feet is equal to 46 dB path loss

240 feet is equal to 52 dB path loss

480 feet is equal to 58 dB path loss – and so on

An example of how this applies with some older equipment is using two Wulfsberg WCS-100 transceivers that both transmit at 43 dBm (20 watts), would present +3 dBm to the other receiver if we had 60 feet between the individual antennas.  Knowing the receiver characteristics, it would require almost 1.0 MHz difference in frequency for co-existence with no degradation.  The Wulfsberg’s performance has been the benchmark for many years.

Most experts agree that the use of as much space as possible between base station antennas provides the most effective insurance against interference.  A properly designed vertically stacked antenna can provide almost 40 dB of isolation (the patterns are within their nulls). This can provide an alternative when horizontal spacing doesn’t permit.

Additionally the selection of the individual frequencies to co-locate plays an important role (keep close-in frequencies as far apart as possible).

### 3.10.2 Cavity Filters

The use of bandpass cavity and notch cavity filters can provide the additional electrical space to allow transmitters and receivers to co-exist when physical space cannot be achieved.  The disadvantage of cavity filters is their size, cost and insertion loss. And at times more than one filter is needed in each antenna path to achieve the necessary isolation to avoid harmful interference.

Figure 3-2, TxRx Cavity with BandPass Elements

Forward Power through the Cavity as measured

on a calibrated Network Analyzer

(Pass 136.975 MHz and Surpress 128-132 MHz)

As can be seen in Figure 3-2 a single bandpass cavity can provide almost 35 dB additional isolation when the frequencies you want to protect are a number of MHz away.  Cascading filters can be used to improve filter action. The summation effect is greater than just adding the results of the two cavities together.

The use of notch filters can greatly improve two close-in frequencies.  At times cascading a notch with a bandpass is necessary.  The major disadvantage other than size and cost is that a notch needs to be inserted in the antenna line for every close-in frequency (station) you need to protect.  Using a notch on the closest frequency and bandpass to protect far away frequencies is used when physical space cannot be achieved.

Figure 3-3, TxRx Cavity with ‘Pass Low’ Element

Forward Power through the Cavity as measured

on a calibrated Network Analyzer

(EXAMPLE: Pass 136.800 MHz and Reject 136.975 MHZ)

As can be seen in Figure 3-3 a single notch cavity filter can provide 25 dB of additional isolation when two stations are only 200 kHz apart.  It also shows that the protection is limited to the one frequency of interest.

### 3.10.3 Ferrite Isolators

Ferrite isolators are commonly used in transmitter combiners in conjunction with cavity filters.  They operate by allowing RF energy to pass effectively in one direction and attenuate the return path to the transmitter. When applied to a single station transceiver the complexity of switching the device in the transmitter path and out of the receive path becomes a problem.  Additionally, the failure mode of this device is such that it is the cause of interference when the transmitter would operate fine without.  These devices are costly to provide and are only used when there is no other solution.

### 3.10.4 Crystal Filters

Crystal filters are an effective way to improve the performance of receivers where the intermodulation is generated within the receiver.  This also helps reduce blocking, desensitization, and cross modulation effects to the protected receiver.  The disadvantages are very high cost, approximately 5 dB additional path loss, need to be inserted in the receive path only (difficult with some transceivers), physically fragile, and easily damaged from nearby lightning strikes.

### 3.10.5 Antenna Gain

At 400 miles the signal from a VHF ground station would be only have a path loss in free space of 131 dB.  This would equate to a signal level at the aircraft of -88 dBm if a line of sight condition existed from a 20 watt transmitter.  This clearly shows that we are limited by the line of sight and not path loss.  Using gain antennas when not necessary can contribute to the interference to nearby stations.  The exception to this statement relates to extended range stations that operate far away from other ground stations.

### 3.10.6 Frequency Change

Frequency change as a solution to an interference problem should only be done when all other possible solutions have been tried and found to be inadequate.   Since there are so many “networked frequencies” this option can be extremely costly since it would also affect stations not located near the interference and the change itself could cause additional interference problems.

## 3.11	Calculating Transmitter Intermodulation Susceptibility

Intermodulation (IM) attenuation is the capability of a transmitter to avoid the generation of signals in the non-linear elements caused by the presence of the carrier and an interfering signal entering the transmitter via the antenna.

It is specified as the ratio, in dB, of the power level of the third order intermodulation product to the carrier power level.  The test is performed with 30 dB of isolation between the test transmitters.

ETSI EN 300 676 section 7.8 defines a test method and sets a limit of 40 dB below the carrier power level.  Most modern transmitters meet or exceed the specified level.  Some of the older transmitters fall short of the specified level.

Below are two examples that identify when additional space isolation is necessary between two transmitters with their antennas are in close proximity.  Normally this space isolation can be obtained by the addition of cavity filters on the offending transmitter. It can also be determined when it’s not necessary to add a cavity filter even though there is a receiver nearby and a mathematical third order product has been identified.

Example 1:

Transmitter ‘A’ - 130.4 MHz, 20 watts (+43 dBm), 60 dB IM attenuation

Transmitter ‘B’ - 131.0 MHz, 7 watts (+38.5 dBm), 40 dB IM attenuation

Physical space between transmitter antennas equals 60 feet (40 dB isolation)

40 dB space minus 30 dB test isolation equals 10 dB additional IM attenuation. Now transmitter ‘A’ has 70 dB and transmitter ‘B’ has 50 dB IM attenuation under this installation.

Third order products –

2F1 - F2  = 129.8 MHz (mostly from transmitter ‘A’)

2F2 - F1 = 131.6 MHz (mostly from transmitter ‘B”)

Checking the airport database identifies stations nearby on both of these frequencies.

129.8 MHz is 1000 feet away (64 dB of path attenuation) identified as ‘C’

131.6 MHz is 2000 feet away (70 dB of path attenuation) identified as ‘D”

Transmitter ‘A’ to station ‘C’

+43 dBm – 70 dB = -27 dBm on 129.8 MHz from antenna ‘A’

-27 dBm – 64 dB (path loss) = -91 dBm signal level at station ‘C’

Transmitter ‘B’ to station ‘D’

+38.5 dBm – 50 dB = -11.5 dBm on 131.6 MHz from antenna ‘B’

-11.5 dBm – 70 dB (path loss) = -81.5 dBm signal level at station ‘D’

This clearly identifies that approximately 10 dB additional isolation is necessary from Transmitter ‘A’ and 20 dB additional isolation is necessary from transmitter ‘B’.

## 3.12	Conclusion

Understanding the transmitter, receiver and transceiver performance related to the interference susceptibility is necessary for co-locating equipment.  Test methods like ETSI EN 300 676 are a good start to insure interference free installations and understanding the limitations involved with co-locating equipment.  Using the lowest power necessary to achieve communications is a good way to work together.  Airports should allow the different airlines their own antenna space, thereby limiting the interference potential to other users, by designing distance between users.  Coordination between the users is also necessary to identify and fix interference issues.

Airports should refrain from designing/building antenna farms which condense or co-locate users to small physical areas, greatly increasing the potential for harmful interference.

# HF Policy

LDOC (LONG DISTANCE OPERATIONAL CONTROL)

HIGH FREQUENCY ASSIGNMENTS

COM CENTER	FREQUENCIES (kHz)

| New York | 3494 | 6640 | 8933 | 11342 | 13348 | 17925 | 21964 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bolivia | 3494 | 6640 | 8933 | 11342 | 13348 | 17925 | 21964 |
| San Francisco | 3494 | 6640 | 8933 | 11342 | 13348 | 17925 | 21964 |
| Barrow | 3494 | 6640 | 8933 | 11342 | 13348 | 17925 | 21964 |
| Guam | 3494 | 6640 | 8933 | 11342 | 11348 | 17925 | 21964 |
| Thailand | 3494 | 6640 | 11342 | 13348 | 17925 | 21964 |  |

FIGURE 4-1

ICAO High Frequency Enroute Radiotelephony Networks

United States

Figure 4-1 (continued)

# Industry Organizations, ASSOCIATIONS and Bodies

## Member Associations

### AIRCRAFT OWNERS AND PILOTS ASSOCIATION

The U.S. Aircraft Owners and Pilots Association (AOPA) was incorporated in New Jersey on May 12, 1939. It was formed to: (1) Promoted, protect, and represent its members and their interests; (2) further art, science, and industry of aeronautics, and (3) promote the economy, safety, and use and popularity of flight in aircraft.

AOPA wanted to gain a voice for general aviation in the International Civil Aviation Organization (ICAO). So, they decided to establish an international general aviation organization, which would be called the International Council of Aircraft Owners and Pilots Association (IAOPA).

IAOPA is a nonprofit federation and has represented international general aviation since October 22, 1964, when the ICAO gave it an official status. ICAO defines general aviation as “All civil aviation operations other than scheduled air services and non-scheduled air transport operations for remuneration or hire.” Currently, 68 IAOPA affiliate organizations represent the interests of more than 470,000 aircraft pilots and owners worldwide for business and personal transportation.

A list of IAOPA Member Organizations can be found on the IAOPA website at http://www.iaopa.org/affil/index.html.

Additional information on AOPA can be found on the AOPA website at http://www.aopa.org .

### NATIONAL BUSINESS AVIATION ASSOCIATION, INC.

The National Business Aviation Association, Inc. (NBAA), is an association of organizations which own or operate aircraft in the conduct of their business.  The Association was formed in 1947 for the purpose of furthering the cause of safety, security and economy of business aircraft operations. The mission statement of NBAA is “To foster an environment that allows business aviation to thrive in the United States and around the World.”

A full membership in NBAA may be accorded to any commercial or industrial enterprise which is engaged in business, commerce, trade or industry and owns and/or operates aircraft of United States registry.  These aircraft are utilized as aids to the conduct of the enterprise's business and are flown by pilots having current professional qualifications of at least a valid commercial license and instrument rating.

The NBAA represents its members at all levels of government, and provides technical information, advice and support for improved equipment and facilities.  The Association also seeks to enhance aviation safety, security and operational economy, and sponsors activities to promote improved understanding of business aviation among all segments of the community.

Additional information on NBAA can be found on the NBAA website at http://www.nbaa.org .

### HELICOPTER ASSOCIATION INTERNATIONAL

The Helicopter Association International (HAI) is an international, independent, non-profit organization.  This Association had its beginning on December 13, 1948 when a group of helicopter operators met in the Los Angeles area and formed an organization known as the California Helicopter Association.  In 1951, due to the growing list of members from other states and Canada, as well as interest from other foreign based operators, the name of the Association was changed to the Helicopter Association of America (HAA).  The Association was incorporated in the State of California in 1954 and became HAI in 1981.

The Operations Department' primary function is to assist owners and operators in enhancing their operations and utilizing their aircraft. Helicopter Association International (HAI) works on behalf of its members on legislative issues that affect operations. HAI coordinates with international, national, and local government authorities, including the Federal Aviation Administration (FAA) and the Transportation Security Administration (TSA), in various matters that impact the businesses of members.

Membership information can be found at HAI’s website at http://www.rotor.com .

Liaison Activities can be found at HAI’s website at http://www.rotor.com .

### HELICOPTER SAFETY ADVISORY CONFERENCE

The Helicopter Safety Advisory Conference (HSAC) was formed in January 1978 after an accident in which 19 people lost their lives when a helicopter came in contact with a platform crane. Realizing there was a lack of communication between oil companies, service organizations, helicopter operations, and the helicopter industry, representatives of these companies agreed to work toward improved safety offshore. The HSAC continues to identify critical issues that affect safety in the Gulf of Mexico and has been recognized both nationally and internationally.

The HSAC has more than 115 members representing major petroleum companies, drilling companies, helicopter operators, oil industry services companies, and helicopter manufacturers. Numerous federal agencies and every branch of the Armed Forces participate in the HSAC. Included in these are the U. S. Army, Air Force, Navy, Coast Guard, Department of Interior, Customs, and the Federal Aviation Administration. The HSAC is a conference and is not incorporated or organized to be a spokesman for the industry. Members are volunteers sharing a common goal.

Additional information on HSAC can be found on the HSAC website at http://www.hsac.org

### NATIONAL AVIATION TRAINING ASSOCIATION

The National Aviation Training Association (NATA) was officially formed on December 28, 1940 with 83 charter member companies. Their goal was to push back the heavy hand of Big Government. NATA was instrumental in lobbying congress and reversing the government’s approach to civilian aviation—if it had not been formed, civilian aviation would have been banned.

Today, NATA lobbies and tracks key legislative and regulatory issues that impact the business operations of FBOs and air charter companies. NATA provides its members with vital information and highly acclaimed publications. It also draws power from its members. And their involvement. Standing committees are made up of experienced and knowable members.

Additional information on NATA can be found on the NATA website at http://www.nata.aero.

## Related Industry Organizations

### ARINC/Rockwell Collins

Aeronautical Radio, Inc. (ARINC) was established in 1929 and is a is a communications company which specializes in providing information handling and processing systems and other communication facilities and services designed specifically to satisfy requirements of the civil aviation community.  ARINC was previously owned by the Carlyle Group, but was sold to Rockwell Collins on December 23, 2013.

Headquarters in Annapolis, Maryland, ARINC employs more than 3200 employees in more than 100 offices around the world providing over 150 value-added solutions and services to customers in 104 countries. To better serve their worldwide customers ARINC has two regional headquarters: Singapore, established in 2003 for the Asia Pacific region, and London, established in 1999 to serve the Europe, Middle East, and Africa region.

More information on ARINC can be found on the Rockwell Collins website at https://www.rockwellcollins.com .

### AIRLINES FOR AMERICA

Formerly known as the Air Transport Association (ATA), Airlines for America (A4A) was founded in 1936 and is the trade and service organization of the scheduled airlines of the United States. A4A promotes the airline industry as the world’s safest form of transportation, advocates for fair taxation and regulations, and coordinates industry actions toward environmental responsibility, economic growth, and technological advances.

A4A is divided into departments which parallel functions of the airlines. The interests of the airlines as an industry are channeled through a system of councils and related committees made up of airline and A4A representatives working together. Departments and Committees of interest to the AFC are the International Affairs Department, the Federal Affairs Department, and the Communications Committee.

Additional information on ATA can be found on the ATA website at http://www.airlines.org.

### THE INTERNATIONAL AIR TRANSPORT ASSOCIATION

The International Air Transport Association (IATA) is a free association of airlines which operate international commercial air services. The association represents some 260 airlines or 83% of total air traffic. The purpose of IATA is to organize cooperation between airlines to enable them to consolidate their experience and requirements. The Association acts as a spokesman for the air transport industry’s relations with governments and governmental organizations. All members of IATA must hold a certificate for scheduled air carriage from a government eligible for membership in the ICAO.

IATA works closely with ICAO and other authorities who are responsible for providing ground radio aids to ensure compatibility and standardization. IATA also works closely with Aeronautical Radio, Inc. (ARINC) and the Societe International de Telecommunications Aeronautiques (SITA).

Additional information on IATA can be found on the IATA website at http://www.iata.org.

### RADIO TECHNICAL COMMISSION FOR AERONAUTICS

The Radio Technical Commission for Aeronautics (RTCA) is a private, not-for-profit corporation that develops consensus-based recommendations regarding communications, navigation, surveillance, and air traffic management (CNS/ATM) system issues. It was founded in 1935 and works in response to requests from the Federal Aviation Administration (FAA). RTCA provides the venue for public-private collaboration that achieves the necessary improvements in the safety and efficiency of the air transportation system.

Through the years, RTCA has received several awards for its service to the aviation community. The organization was awarded the 1949 Collier Trophy for "A guide plan for the development of a system of air navigation and traffic control for safe and unlimited aircraft operations under all weather conditions." Additionally, in 1994, the FAA named RTCA, Inc. as the U.S. recipient of the ICAO 50th anniversary Medal of Honour. This unique recognition identified RTCA as the single most important U.S. contributor organization to the advancement and support of civil aviation since the creation of ICAO by the Chicago Convention in 1944.

Additional information on RTCA can be found on the RTCA website at http://www.rtca.org .

### AEROSPACE AND FLIGHT TEST RADIO COORDINATING COUNCIL

The Aerospace and Flight Test Radio Coordinating Council (AFTRCC) was organized in 1954 and is composed of major aerospace manufacturers who represent the aerospace manufacturing industry for flight test voice and telemetry frequencies.  The AFTRCC coordinates on usage of flight test spectrum by both government agencies (military and non-military) and non-government aerospace manufacturing entities.

Flight test communications are those nonpublic voice and telemetry communications required to support and coordinate the flight testing of aerospace vehicles and their major components.  These tests are essential in the development of aircraft and space vehicles to obtain comprehensive information on the actual performance characteristics of the vehicle.  The four basic types of flight testing are: (1) Experimental and certification flight testing of new aircraft design; (2) Production flight testing of newly manufactured or modified aircraft; (3) Flight testing of major components; and, (4) customer crew training flight test.  The primary purpose of flight test communications is to assure the safety of flight test activities.  Safety considerations require reliable voice and telemetry communications links between the crew of the test vehicle, engineering personnel and test observers in supporting aircraft.  In addition, flight test communications are required to collect test data.  The most practicable means of gathering the test data is to telemeter it from test vehicles to ground tracking stations.  Another important role of flight test communications support, especially telemetry, is to increase the efficiency of flight testing.  With adequate communications support, tests can be modified while in progress as necessary to gather additional or different test data.  Flight test telemetry and voice communications requirements are technically and operationally distinct.

Additional information on AFTRCC can be found on the AFTRCC website at http://www.aftrcc.org

### SOCIETE INTERNATIONALE de TELECOMMUNICATIONA AERONAUTIQUES

Societe International de Telecommunications Aeronautiques (SITA) was founded in 1949 and is a partner with airlines, airports, and many related air transport organizations. SITA is owned by the community, with a Board of Directors comprising shareholders and customers from across the transport industry. The organization aims to provide innovative and community-focused solutions that offer the industry greater cost-effectiveness. SITA has over 600 members and around 1800 customers in over 220 countries and territories. SITA is also the world’s leading provider of IT business solutions and communication services to the air transport industry.

Additional information on SITA can be found on the SITA website at http://www.sita.aero.

### REGIONAL AIRLINE ASSOCIATION

The Regional Airline Association (RAA) represents U.S. regional and commuter air carriers in various National activities. The RAA monitors legislative activity at various governmental levels which may have a potential impact on regional and/or commuter airlines.  The RAA advises its members of these activities, and participates in the consideration of associated legislation at the Federal, State and local levels. It also schedules four Operations Conferences each year that address safety, air traffic control, training programs and other similar subjects and are used to discuss operational problems with representatives of the FAA, the National Transportation Safety Board, and certificated air carriers.

Regular members of the RAA consist of regional and commuter air carriers, as defined by Civil Aeronautics Board (CAB), which are engaged in the air carriage of passengers, cargo and/or mail on a scheduled basis, and certificated air carriers who are engaged in operations normally performed by commuter airlines.  Regular membership also includes "non-operating air carriers."  Associate Members consist of persons, companies, and organizations engaged in activities related either directly or indirectly to commercial aviation.  Associate Members are eligible to participate in affairs of the Association except they are not eligible to vote.  Affiliate members are restricted to colleges and universities (or members of their faculties), state and local governments and state aviation associations.  Affiliate members are not eligible to vote in RAA proceedings.

Additional information on RAA can be found on the RAA website at http://www.raa.org.

## Federal and International Agencies

### THE FEDERAL COMMUNICATIONS COMMISSION

The Federal Communications Commission (FCC) is an independent United States government agency, directly responsible to Congress. The FCC was established by the Communications Act of 1934 and is charged with regulating interstate and international communications by radio, television, wire, satellite and cable. The FCC's jurisdiction covers the 50 states, the District of Columbia, and U.S. possessions.

The FCC is directed by five Commissioners appointed by the President and confirmed by the Senate for 5-year terms, except when filling an unexpired term. The President designates one of the Commissioners to serve as Chairperson. Only three Commissioners may be members of the same political party. None of them can have a financial interest in any Commission-related business.

As the chief executive officer of the Commission, the Chairman delegates management and administrative responsibility to the Managing Director. The Commissioners supervise all FCC activities, delegating responsibilities to staff units and Bureaus.

The Commission staff is organized by function. There are seven operating Bureaus and ten Staff Offices. The Bureaus’ responsibilities include: processing applications for licenses and other filings; analyzing complaints; conducting investigations; developing and implementing regulatory programs; and taking part in hearings. Even though the Bureaus and Offices have their individual functions, they regularly join forces and share expertise in addressing Commission issues.

Additional information on the FCC can be found on the FCC website at https://www.fcc.gov/ .

### THE FEDERAL AVIATION ADMINISTRATION

The Federal Aviation Administration (FAA) is responsible for the safety of civil aviation. The Federal Aviation Act of 1958 created the agency under the name Federal Aviation Agency. The FAA adopted its present name in 1967 when it became a part of the Department of Transportation. FAA major roles include:

Regulating civil aviation to promote safety

Encouraging and developing civil aeronautics, including new aviation technology

Developing and operating a system of air traffic control and navigation for both civil and military aircraft

Researching and developing the National Airspace System and civil aeronautics

Developing and carrying out programs to control aircraft noise and other environmental effects of civil aviation

Regulating U.S. commercial space transportation

An Administrator manages FAA, assisted by a Deputy Administrator. Five Associate Administrators report to the Administrator and direct the line-of-business organizations that carry out the agency's principle functions. The Chief Counsel and nine Assistant Administrators also report to the Administrator. The Assistant Administrators oversee other key programs such as Human Resources, Budget, and System Safety.

Additional information on the FAA can be found on the FAA website at http://www.faa.gov .

### NATIONAL TELECOMMUNICATIONS AND INFORMATION ADMINISTRATION

The National Telecommunications and Information Administration (NTIA), created in 1978, is

the President's principal adviser on telecommunications and information policy issues, and in this role frequently works with other Executive Branch agencies to develop and present the  Administration's position on these issues.

In addition to representing the Executive Branch in both domestic and international

telecommunications and information policy activities, NTIA also manages the Federal use of

spectrum; performs cutting-edge telecommunications research and engineering, including

resolving technical telecommunications issues for the Federal government and private sector;

and administers infrastructure and public telecommunications facilities grants.

A fundamental mission of NTIA is to promote market-based policies which lower prices to

consumers and encourage innovation, while harnessing the resources of the Federal government to support spectrum-based technologies which enhance efficiency and productivity.

Additional information on the NTIA can be found on the NTIA website at http://www.ntia.doc.gov

### INTERNATIONAL CIVIL AVIATION ORGANIZATION

The foundation of the International Civil Aviation Organization (ICAO) was laid at the International Civil Aviation Conference held in Chicago in November 1944.  At that time, an International Civil Aviation and International Air Transport Agreement, an International Air Services Transit Agreement and a Convention on International Civil Aviation were all adopted.  The latter culminated in formation of the ICAO on April 4, 1947.  Since that time, ICAO has been responsible for obtaining international agreement and implementing numerous standards, practices and procedures.

Additional information on the ICAO can be found on the ICAO website at http://www.icao.int .

### THE INTERNATIONAL TELECOMMUNICATION UNION

The purposes of the International Telecommunication Union (ITU) are maintaining and extending international cooperation for the improvement and rational use of telecommunications, prompting the development of technical facilities and their most efficient operation with a view of improving the efficiency of telecommunication services, increasing their usefulness, making them generally available to the public and harmonizing the actions of nations in the attainment of common goals.

The organization of the ITU consists of the Plenipotentiary Conference, ITU Council, General Secretariat, and Bureaus of Development, Radiocommunications, and Telecommunications Standardization. It is based in Geneva, Switzerland, and its membership includes 191 Member States and more than 700 Sector Members and Associates.

Additional information on the ITU can be found on the ITU website at http://www.itu.int .

# Frequency Allocations, Coordination and Assignments

## The Usable Spectrum

Since the discovery of electromagnetic waves, man has continually expanded the usable area of spectrum, and, almost simultaneously, developed the necessary technology and demand to saturate it. From the starting point near 1 kHz, frequency utilization below 10 kHz and up to 300 GHz and beyond has been achieved. This area, of course, is only a small fraction of the entire spectrum.  There are two major reasons why frequency usage is concentrated in such a small area. One is man's inability to produce, manipulate, transmit and receive frequencies across the spectrum with equal ease and reliability. Consequently, frequency usage is usually confined to those frequency bands that allow easy, economical and stable operations. The second reason for this concentration is the demand for the varying, unique propagation characteristics inherent to certain frequency bands.  The latter limitation is more restrictive because it is imposed by nature. Increased technology will gradually negate the former. The highest level of utilization is in portions commonly referred to as the radio frequency spectrum which encompasses frequencies from 3 kHz to 300 GHz.

The spectrum is divided by wavelength into bands between each power of 10. For example, wave lengths 104  and 105  meters constitute the very low frequency (VLF) band and include the frequencies from 3-30 kHz (See Figure 8-1). Due to unique propagation characteristics, some frequency bands are in greater demand.  The high frequency (HF) area, for example, is the only portion of the spectrum capable of providing relatively reliable, long-range propagation which is capable of accommodating voice or teletype data. Hence, the HF spectrum is the only method of communicating over long distances from one point on earth to another without utilizing a relay or connecting cable. Accordingly, the band is congested with heavy overcrowding, other frequency bands present similar problems.

Changes in technological, political and socioeconomic factors have been responsible for the increased demand for spectrum. These changing factors are frequently interrelated and often have a subtle, but very significant, long-term impact on frequency usage.  Socioeconomic and political factors also effect the demand for frequencies; e.g., the rapidly expanding world population.  The population growth is continually increasing the requirements for additional communications capability, and, consequently, causing additional facilities and systems to be created. The world's changing economy also contributes to the increased demands for communications systems.  As the standard of living increases, the demand for state-of-technology products increases at a commensurate pace.  For example, the number of available channels in the VHF aeronautical frequency band has been doubled by reducing the channel bandwidth from 50 kHz to 25 kHz. (The current U.S./Canada Interim Channeling Arrangement for 25 kHz channels in the frequency band 128.8125 - 132.0125 MHz is provided in Chapter 2). In addition, new, developing nations are emerging, and as these governments become stronger and more sophisticated, they logically demand a fair share of the spectrum to serve their national interests.

Rapidly increasing technology has also contributed to the frequency demand/availability problem.  The achievements in space technology, for example, have had a profound effect on frequency usage and planning. Bands of frequencies formerly used for short-range communications are now being used for an earth satellite relay system, thereby requiring a costly conversion to accommodate the former users. The same relay system, however, is relieving the over taxed earth relay, cable and HF systems of long distance communications circuits. Intensive, large scale research is on the verge of making possible an entirely new concept of communications. The use of fiber-optics transmission systems as an alternative to free-space may make available a spectral area several thousand times larger than all the frequency bands presently in use under 300 GHz. While a system of such enormous potential will serve to alleviate many frequency problems, the additional demands it will trigger will undoubtedly create new and perhaps more complex problems for the frequency management community.

### 8.1.1	Problems Resulting From Increased Frequency Usage

Presently, nearly all portions of the usable spectrum are assigned for use. Most countries, especially the emerging nations, have become increasingly aware of the advantages, and, to a lesser degree, the problems associated with exploiting the frequency spectrum. However, these problems are not limited to the developing nations. On the contrary, all of the highly industrialized countries, enjoying their high standards of living, are experiencing similar difficulties.  It is especially in these countries of high and growing frequency usage that proper attention to frequency management is paramount. In the United States, the rapidly escalating frequency usage has focused high-level attention on several resultant problems. For example, electromagnetic phenomena, either directly or indirectly, can degrade the performance of an electronic receiver or system. The degradation may be caused by a variety of conditions, which may be difficult to correct without restricting the operations of those involved. A great deal of effort is currently being expended to resolve these interference problems.

| Band Number | Symbols | Frequency Range (lower limit exclusive, upper limit inclusive) | Corresponding Metric Subdivision | Metric Abbreviations for the Bands |
| --- | --- | --- | --- | --- |
| 4 | VLF | 3 to 30 kHz | Myriametric waves | B.Mam |
| 5 | LF | 30 to 300 kHz | Kilometric waves | B.km |
| 6 | MF | 300 to 3000 kHz | Hectometric waves | B.hm |
| 7 | HF | 3 to 30 MHz | Decametric waves | B.dam |
| 8 | VHF | 30 to 300 MHz | Metric waves | B.m |
| 9 | UHF | 300 to 3000 MHz | Decimetric waves | B.dm |
| 10 | SHF | 3 to 30 GHz | Centimetric waves | B.cm |
| 11 | EHF | 30 to 300 GHz | Millimetric waves | B.mm |
| 12 |  | 300 to 3000 GHz | Decimillimetric waves |  |

NOMENCLATURE OF FREQUENCY BANDS

FIGURE 8-1

## 8.2.	ELECTROMAGNETIC PROBLEMS

Electromagnetic problems may be categorized into two distinct areas: (1) Electromagnetic Interference (EMI); and, (2) Electromagnetic Compatibility (EMC).  EMI is an electromagnetic phenomenon which either directly or indirectly can contribute to degradation in the performance of an electronic receiver or system.  EMI can be caused by a large variety of conditions, many of which are difficult to correct without restricting the operations of those involved.  A great deal of effort is currently being expended in the resolution of these interference problems.

EMC is the capability of electronic equipment and systems to operate in the intended environment at designated levels of efficiency without degradation due to unintentional interference.  The continually increasing utilization of frequency-employing devices, and the consequent increase in EMI problems, have clearly established the necessity for careful planning prior to activating electronic systems.

## 8.3.	THE NEED FOR MANAGEMENT, CONTROL AND COOPERATION

With an increasing demand for frequencies and a fixed supply, the necessity for an efficient regulatory system is obvious.  Without such a system, the resulting EMI would effectively deny the use of the spectrum to many and encourage the employment of more powerful transmitters, which, in turn, would produce more EMI.  To be effective, the system requires the following elements:

### 8.3.1	Central Management

To preclude a chaotic condition and to obtain maximum utility from the spectrum, an orderly system of distributing frequencies among competing users is imperative.  Ideally, this system would equitably divide the usable spectrum, set priorities, establish rules, plan for future changes, and, in general, promote the most efficient use of the spectrum possible.  In addition, the system must allow for periodic review and adjustment to changing conditions.

### 8.3.2	Means of Control

Agencies administering the frequency spectrum must have the authority to approve or disapprove requests for frequency usage, as well as to enforce the rules governing frequency deviation, power limits, bandwidth and interference criteria.  A method must exist to resolve EMI problems.  In addition, agencies and user-community must be able to represent their respective sectors, both nationally and internationally.

### 8.3.3	User Cooperation

Because electromagnetic radiation does not respect national boundaries, successful worldwide frequency management requires the unanimous cooperation of all nations.  Consequently, national frequency management must also have cooperation from all users, as a single entity or agency cannot police the entire spectrum on a national scale.

## 8.4.	DISTRIBUTION OF FREQUENCIES

Frequency management includes the administration and control of electromagnetic radiation to permit the orderly, efficient, and effective use of the spectrum.  The distribution of frequencies or bands of frequencies consists of three distinct processes: (1) Allocation; (2) Allotment; and, (3) Assignment.  Frequency allocation is the designation of frequency bands for performing specific functions or services.  Allocations are made to communication services such as fixed, mobile, aeronautical, etc.  The various telecommunication propagation modes are depicted in Figure 8-10.  Frequency allotment is the designation of specific frequency bands or discrete frequencies (within a prescribed allocation) for use by specific countries or within certain areas.  Within the United States, allotments are made to specific agencies or activities.  Frequency assignment is the process of designating a specific frequency for use at a particular station under specified operating conditions, subject to limitations and/or restrictions of both national and international frequency allocations for a specific service.  The following major types of services are considered to be of interest to the AFC membership:

### 8.4.1	Fixed Service

Fixed service is defined as radio communications between specified fixed locations.  The word "specified" is used to establish the difference between fixed service and certain broadcast services.  Fixed service includes all communications points and through relays along established communications routes.

### 8.4.2	Mobile Service

Mobile service relates to radio communication between mobile stations or between mobile stations and land stations.  Aeronautical mobile service includes radio communication between aircraft, or between aircraft and aeronautical stations; e.g., air traffic control facilities and the ARINC LDOCF.  Aeronautical mobile services are normally identified in frequency allocation charts and tables by the letter "R" or letters "OR".  The HF frequency bands allocated to the aeronautical mobile (R) service are identified in Figure 8-2.

### 8.4.3	Radio Navigation Service

Frequencies have been allocated in various parts of the spectrum for aeronautical navigation aids.  Specific frequency bands have been allocated for VHF Omnidirectional Radio Range (VOR), Tactical Air Navigation (TACAN), and Instrument Landing System (ILS) facilities.

### 8.4.4	Space Service

The space service includes such functions as Space Research Satellite, Intersatellite and Communications Satellite services.  A typical space system include the fixed terrestrial stations associated with a particular service.  In addition, satellite services can augment almost any other radio service; e.g., the NAVSTAR Global Positioning System Radionavigation Satellite System augments the TACAN system.

### 8.4.5	Distress and Emergency Service

Frequencies have been designated in many frequency bands throughout the spectrum for distress, emergency and survival purposes.  The international aeronautical emergency and distress frequencies are described in Figure 8-3.  In addition to the frequencies identified in Figure 8-3, the carrier frequencies 3023 and 5680 kHz may be used by mobile stations for intercommunication between mobile stations engaged in coordinated search and rescue operations.  The frequency 123.1 MHz may be used by mobile stations and by land stations directly associated with such mobile stations, for search and rescue communications at the scene of an incident.

FIGURE 8-2

HF SERVICE RANGES.  AERONAUTICAL MOBILE (R) SERVICE

| Frequency Bands Allocated to the (R) Service | Service Day | Ranges Night | Interference (15 dB) Day | Ranges Night |
| --- | --- | --- | --- | --- |
| kHz | km | km | km | km |
| 2850 - 3025 | 100 | 500 | 700 | 3,500 |
| 3400 - 3500 | 100 | 800 | 700 | 4,000 |
| 4650 - 4700 | 350 | 1,400 | 1,200 | 5,500 |
| 5450 - 5480 * | 450 | 1,800 | 1,500 | 6,500 |
| 5480 - 5680 | 450 | 1,800 | 1,500 | 6,500 |
| 6525 - 6685 | 650 | 2,200 | 1,900 | 3,000 |
| 8815 - 8965 | 1,000 | 3,400 | 3,800 | 11,000 |
| 10005 - 10100 | 1,250 | - | 5,500 | - |
| 11275 - 11400 | 1,500 | - | 6,000 | - |
| 13260 - 13360 | 1,900 | - | 7,700 | - |
| 17900 - 17970 | 2,600 | - | 10,000 | - |
| 21924 - 22000 | 2,600 + | - | 10,000 + | - |

*In ITU Region 2 Only.

FIGURE 8-3

INTERNATIONAL AERONAUTICAL EMERGENCY

AND DISTRESS FREQUENCIES

| FREQUENCY | SERVICE | FUNCTION |
| --- | --- | --- |
| 500 & 2182 kHz | Aeronautical, Maritime, Survival Craft | Distress |
| 4125 kHz | Aeronautical, Maritime Survival Craft | Supplement to 2182 kHz |
| 121.54 MHz | Aeronautical Emergency | Emergency and Safety |
| 123.1 MHz | Aeronautical Auxiliary | Emergency and Safety |
| 243 MHz | Aeronautical (Military) | Emergency and Survival |
| 406.028 MHz | Aeronautical Emergency | Emergency and Safety |

## 8.5.	AIR TRAFFIC CONTROL FREQUENCIES

The purpose of air traffic control (ATC) facilities, of course, is to control the movement of aircraft taxiing, departing and approaching air terminals and enroute in controlled air space.  ATC facilities require frequencies that are instantly available, free from interference and not subject to interruptions to insure the safe, orderly and expeditious control of air traffic.  This capability is provided by assigning frequencies to ATC facilities which are used exclusively for ATC purposes.

## 8.6.	NAVIGATIONAL AID FREQUENCIES

The frequencies allocated to the functions of aeronautical navigation aids are described in the following:

### 8.6.1	Instrument Landing System (ILS)

Localizer and glideslope frequency pairs have been standardized by the International Civil Aviation Organization (ICAO) to form 20 ILS channels.  The 75 MHz fan marker frequency is used by all agencies, civil and government, for ILS marker beacons.

### 8.6.2	Tactical Air Navigation (TACAN)

The TACAN system operates on 126 frequency channels spaced 1 MHz apart.  The ground station transmitters operate in the frequency ranges of 962 to 1024 MHz and 1151 to 1213 MHz.  The airborne transmitters operate in the frequency range of 1025 to 1150 MHz.  With few exceptions, only the ground transmit frequency is assigned and recorded in the frequency records; use of the paired airborne transmit frequency is assumed.  An exception to the foregoing is developmental or experimental operations when the TACAN is not used in the normal manner.  TACAN channels 1 through 16 and 60 through 69 are generally used for military tactical purposes.  The remaining 100 channels are used in the common civil and military national airspace system.  TACAN channel 98 is normally reserved for equipment testing.

### 8.6.3	VHF Omnidirectional Range (VOR)

Frequencies ending in even tenth decimals between 108.2 and 112.0 MHz are reserved for the VOR.  In addition, all frequency channels, 100 kHz spaced from 112.0 through 117.9 MHz, are reserved for VOR to make a total of 79 VOR channels.  The 108.0 MHz frequency is reserved for VOR testing.

### 8.6.4	TACAN and VOR/ILS Paired Frequencies

When a TACAN operates with a VHF navigational facility (VOR or ILS), the transponder is collocated and the frequency paired with that of the VHF facility.  If the system is used for terminal services such as airport approach or landing, the facilities are considered to be collocated when the transponder and VHF antennas are not more than 260 feet apart.  For enroute procedures, collocation exists if the antenna separation does not exceed 2,000 feet.  Where the separation exceeds these figures, a VOR/ILS frequency from one pair and the TACAN frequency from another pair are assigned.  This alerts users to the fact that they are not receiving azimuth and range information from the same point.

### 8.6.5	ATCRBS/IFF/SIF

The Air Traffic Control Radar Beacon System (ATCRBS) Identification, Friend or Foe (IFF)/Selective Identification Features (SIF) operates on standard transmit frequencies of 1030 MHz for the ground interrogators and 1090 MHz for airborne transponders.  Only the ground station frequency 1030 MHz is normally recorded in frequency assignment records.  The FAA has statutory responsibility for the ATCRBS.

## 8.7.	STANDARD FREQUENCY AND TIME BROADCASTS

Frequencies have been nationally and internationally allocated and assigned to designated radio stations to provide an accurate source of calibrating frequency sensitive equipment.  During the transmission of time signals, stations are prohibited from any transmissions that might interfere with the reception of these signals by other stations.  Key points concerning the national and international standard broadcasts are contained in the following paragraphs.

### 8.7.1	U.S. Standard Broadcasts

The National Bureau of Standards (NBS) operates two HF radio stations - WWV near Fort Collins, Colorado and WWVH at Kihei, Maui, Hawaii.  Each station broadcasts standard frequency and time signals of high accuracy.  The coordinates of WWV are 40o 40' 49" north latitude and 105o 02' 27" west longitude.  The coordinates of WWVH are 20o 46' 02" north latitude and 156o 27' 42" west longitude.  Both stations transmits simultaneously on the same frequencies.  The transmitted frequencies of both stations are accurate to within one part in 10 billion and are broadcast on 2500, 5000, 10000, 15000, 20000 and 25000 kHz.  The NBS also operates a low frequency (60 kHz) station, WWVB, near Fort Collins, Colorado.  These stations are used to coordinate operation of the global networks of missile and satellite stations, to assist other Government efforts which require accurate time and frequency, to improve the uniformity of frequency measurement on a national and international basis, and to provide a more accurate standard of frequency for electronic research and development.

### 8.7.2	Foreign Standard Broadcasts

The Canadian Dominion Observatory (Ottawa, Ontario) broadcasts standard frequency and time signals continuously over station CHU on frequencies 3330, 7335 and 14670 kHz.  In addition, the Tokyo Astronomical Observatory broadcasts standard time and frequency signals over station JJY on 2500, 5000, 10000 and 15000 kHz.

## 8.8.	INTERNATIONAL DISTRESS AND EMERGENCY FREQUENCIES

The U.S. Government has adopted the international distress and emergency frequencies shown in Figure 8-3 and discussed in paragraph 8.4.5. (Note: Only those frequencies relative to the aeronautical industry are depicted in the chart).

## 8.9.	NAVIGATIONAL AID IDENTIFIERS

Navigational aids are assigned unique codes to identify their location.  Various types of navigational aid identifiers are assigned to facilities as follows:

### 8.9.1	Airports, Heliports and Seaplane Bases

Three-letter identifiers are assigned to those airports, heliports and seaplane bases on which there is a manned FAA air traffic control facility or a terminal air navigational aid within the airport boundaries, or to those which receive Department of Defense airlift service or scheduled tour air carrier services directly into the airport.  United States airports, heliports and seaplane bases not meeting the requirements stated above may be assigned a one-letter, two-numeral identifier, provided such landing facilities average at least two general aviation inbound flight plans daily.

### 8.9.2	Instrument Landing System Localizers and Associated DMEs

The initial Instrument Landing System (ILS) Localizer is assigned the same identifier as the airport which it serves, even though the other compass locator is given a location name different from the city or airport name.  If additional ILS facilities are established, separate three-letter identifiers are assigned to each facility.  The foregoing applies to facilities when an additional ILS serving the opposite end of an ILS-equipped runway, to a parallel runway even though both systems operate on the same frequencies on a non-simultaneous basis, and two different type localizer equipment serving the same runway.  To distinguish the ILS localizer from another VHF facility using the same identifier, the keyed ILS identifier is preceded by the letter "I".  When the DME is installed as an integral part of an ILS, the same identifier, preceded by the letter "I", is assigned to both the localizer and DME.

### 8.9.3	Interim Standard Microwave Landing System

The criteria used to assign an identifier to an ILS also applies to the Interim Standard Microwave Landing System (ISMLA).  The identifier assigned, however, is preceded by the letter "M".  If an ILS and ISMLS are installed to serve the same runway end, one basic identifier may be assigned, preceded by an "I" for the ILS and an "M" for the ISMLS.

### 8.9.4	Compass Locators (COMLO)

The COMLO is a nondirectional radio beacon (NDB), usually of low power, strategically located on an ILS approach path to provide L/MF azimuth guidance to an airport, in addition to the more precise guidance of the ILS LOC.  COMLO's are normally collocated with ILS Outer Markers (OM) and/or Middle Markers (MM), and referred to as "LOM" and "LMM," respectively.  The Morse code identification is derived from the three-letter identifier of the ILS.  The LOM will be the first two letters, while the LMM will be the last two letters, with the middle letter overlapping.  For example, COMLO's installed with the Los Angeles, CA (LAX) ILS would be "LA" for the LOM and "AX" for the LMM.  To the extent possible, the frequencies of the LOM and LMM shall not be separated less than 15 kHz nor more than 25 kHz and shall be in descending frequency order.

### 8.9.5	Nondirectional Radio Beacons

The NDB is a free-standing nondirectional radio beacon designed to provide navigational service over a specified radial distance from the facility.  It can have power from 10 W to 1000 W, typically 25 W, depending on the need.  It should be noted that due to very heavy congestion in this band, the FAA Spectrum Management Office shall do everything possible, by coordinating with Air Traffic Control (AT) and Flight Service (FS), to engineer the lowest possible emitted power to cover the requirement.  Its identification is by three letters and is chosen by AT.

### 8.9.6	VOR, TACAN and VORTAC

VORs, TACANs and VORTACs are assigned three-letter identifiers based on chart names.  If two or more such facilities are established at a location, they are assigned different names and identifiers.

## 8.10.	BUSINESS RADIO FREQUENCIES ALLOCATED FOR AVIATION TERMINAL USE (12.5 kHz channel spacing)

The ATU channels are identified in Figure 8-4.

| BASE/MOBILE | MOBILE |
| --- | --- |
| 460.6500 | 465.6500 |
| 460.6625 | 465.6625 |
| 460.6750 | 465.6750 |
| 460.6875 | 465.6875 |
| 460.7000 | 465.7000 |
| 460.7125 | 465.7125 |
| 460.7250 | 465.7250 |
| 460.7375 | 465.7375 |
| 460.7500 | 465.7500 |
| 460.7625 | 465.7625 |
| 460.7750 | 465.7750 |
| 460.7875 | 465.7875 |
| 460.8000 | 465.8000 |
| 460.8125 | 465.8125 |
| 460.8250 | 465.8250 |
| 460.8375 | 465.8375 |
| 460.8500 | 465.8500 |
| 460.8625 | 465.8625 |
| 460.8750 | 465.8750 |
| 460.8875 | 465.8875 |

FIGURE 8-4.  ATU CHANNELS

FIGURE 8-5

FREQUENCY AND WAVELENGTH RELATIONSHIPS

FIGURE 8-6

POWER/VOLTAGE CONVERSATION TABLE

FIGURE 8-7

RADIO AND OPTICAL LINE-OF-SITE DISTANCES

FIGURE 8-8

GEOGRAPHICAL DIVISIONS USED FOR

INTERNATIONAL FREQUENCY ALLOCATIONS

FIGURE 8-9

NATIONAL FREQUENCY COORDINATION AND ASSIGNMENT PROCESS

FIGURE 8-10

TELECOMMUNICATIONS PROPAGATION MODES

# Glossary, Two Letter State and Territory Abbreviations, Terms and Definitions

## GLOSSARY OF ABBREVIATIONS AND ACRONYMS

| * A * |  |  |  |
| --- | --- | --- | --- |
| AAS | Airport Advisory Service |  |  |
| AASR | Airport & Airways Surveillance Radar |  |  |
| ACARS | Aircraft Communications Addressing & Reporting System |  |  |
| ACC | Area Control Center |  |  |
| ACCTS | Airlines Coordinating Committee for Telecommunications Service |  |  |
| ADF | Automatic Direction Finder |  |  |
| ADIS | Automated Data Interchange System |  |  |
| ADIZ | Air Defense Identification Zone |  |  |
| ADS | Automatic Dependent Surveillance |  |  |
| ADS | B | Automatic Dependent Surveillance | Broadcast |
| AECS(P) | Aeronautical Emergency Communications System Plan |  |  |
| AEEC | Airlines Electronic Engineering Committee |  |  |
| AES | Aeronautical Enroute Service |  |  |
| AFC | Aeronautical Frequency Committee |  |  |
| AFI | Africa | Indian Ocean |  |
| AFS | Aeronautical Fixed Service |  |  |
| AFTN | Aeronautical Fixed Telecommunication Network |  |  |
| AFTRCC | Aerospace & Flight Test Radio Coordinating Council |  |  |
| AIM | Airman's Information Manual |  |  |
| AIREPS | Inflight Weather Reports |  |  |
| AIS | Aeronautical Information Service |  |  |
| ALCAC | Air Lines Communication Advisory Council |  |  |
| ALPA | Air Line Pilots Association |  |  |
| AMC | Avionics Maintenance Conference |  |  |
| AME | Amplitude Modulation Equivalent |  |  |
| AMS | Aeronautical Mobile Service |  |  |
| ANC | Air Navigation Commission/Air Navigation Conference |  |  |
| ANP | Air Navigation Plan |  |  |
| AOC | Aeronautical Operational Control |  |  |
| AOPA | Aircraft Owners & Pilots Association |  |  |
| APC | Area Positive Control |  |  |
| ARINC | Aeronautical Radio Incorporated |  |  |
| ARSR | Air Route Surveillance Radar |  |  |
| ARTCC | Air Route Traffic Control Center |  |  |
| ARTS | Automated Radar Traffic Control Center |  |  |
| ASRI | Aviation Spectrum Resources Incorporated |  |  |
| ASTRA | Application of Space Techniques Relating to Aviation |  |  |
| ATA | Air Transport Association of America |  |  |
| ATC | Air Traffic Control |  |  |
| ATCRBS | Air Traffic Control Radar Beacon System |  |  |
| ATIS | Automated Terminal Information Service |  |  |
| ATMAC | Air Traffic Management Advisory Committee |  |  |
| ATS | Air Traffic Services |  |  |
| ATU | Aviation Terminal Use |  |  |
| AVINET | ARINC Data Network Services |  |  |
| AVPAC | Aviation Packet, bit oriented protocol |  |  |
| AWARC | Aeronautical World Administrative Radio Conference |  |  |
| * B * |  |  |  |
| BCN | Beacon |  |  |
| BDT | Telecommunication Development Bureau |  |  |
| * C * |  |  |  |
| CAA | Civil Aviation Authority |  |  |
| CAB | Civil Aeronautics Board |  |  |
| CAR | Caribbean/Civil Air Regulations |  |  |
| CARERI | Chinese Aeronautical Radio Electronics Research Institute |  |  |
| CAS | Collision Avoidance System |  |  |
| CCIR | International Radio Consultative Committee | Now ITU | R |
| CCITT | International Telegraph & Telephone Consultative Committee | Now ITU | T |
| CEP | Central East Pacific |  |  |
| CNS/ATM | Communications, Navigation, Surveillance/Air Traffic Management |  |  |
| COM/OPS | Communications/Operations |  |  |
| COMAC | Communications Advisory Committee (IATA) |  |  |
| COMLO | Compass Locations |  |  |
| CRAF | Civil Reserve Air Fleet |  |  |
| CTA | Control Area |  |  |
| CWP | Central West Pacific |  |  |
| * D * |  |  |  |
| DF | Direction Finder |  |  |
| DGPS | Differential GPS |  |  |
| DME | Distance Measuring Equipment |  |  |
| DSB | Double Side Band |  |  |
| DVOR | Doppler VOR |  |  |
| * E * |  |  |  |
| EAEC | European Airlines Electronic Committee |  |  |
| EARC | Extraordinary Administrative Radio Conference |  |  |
| EHF | Extremely High Frequency |  |  |
| ELT | Emergency Locator Transmitter |  |  |
| EMC | Electromagnetic Compatibility |  |  |
| EMI | Electromagnetic Interference |  |  |
| ERP | Effective Radiated Power |  |  |
| ESRO | European Space & Research Organization |  |  |
| ETSI | European Telecommunications Standards Institute |  |  |
| EUM | European/Mediterranean |  |  |
| EUR | Europe |  |  |
| EUROCAE | European Organization for Civil Aviation Electronics |  |  |
| * F * |  |  |  |
| FA | Fixed Aeronautical (Ground/Air Station Class) |  |  |
| FAA | Federal Aviation Administration |  |  |
| FAR | Federal Aviation Regulations |  |  |
| FCC | Federal Communications Commission |  |  |
| FE | Far East |  |  |
| FIC | Flight Information Center |  |  |
| FIO | Flight Information Office |  |  |
| FIR | Flight Information Region |  |  |
| FMAC | Frequency Management Advisory Council (NTIA) |  |  |
| FMSG | Frequency Management Study Group (ICAO) |  |  |
| FSK | Frequency Shift Keying |  |  |
| FSS | Flight Service Station |  |  |
| FX | Fixed Service Station Class |  |  |
| * G * |  |  |  |
| G/A | Ground | to | Air |
| GHZ | Gigahertz |  |  |
| GMT | Greenwich Mean Time |  |  |
| GNSS | Global Navigation Satellite System |  |  |
| GPS | Global Positioning System |  |  |
| GS | Glide slope |  |  |
| GSARS | Ground Station Automated Reporting System |  |  |
| GWARC | General World Administrative Radio Conference |  |  |
| * H * |  |  |  |
| HAA | Helicopter Association of America |  |  |
| HAI | Helicopter Association International |  |  |
| HF | High Frequency |  |  |
| HSAC | Helicopter Safety Advisory Conference |  |  |
| HZ | Hertz (Cycles per second) |  |  |
| * I * |  |  |  |
| IAOPA | International Council of Aircraft Owners & Pilots Association |  |  |
| IATA | International Air Transport Association |  |  |
| IBAC | International Business Aviation Council, Ltd. |  |  |
| ICAO | International Civil Aviation Organization |  |  |
| ICT | Information and Communication Technologies |  |  |
| IFALPA | International Federation of Air Line Pilots Association |  |  |
| IFF | Identification, Friend or Foe |  |  |
| IFR | Instrument Flight Rules |  |  |
| IFRB | International Frequency Registration Board |  |  |
| IFSS | International Flight Service Station |  |  |
| IGA | International General Aviation |  |  |
| IGIA | Interagency Group on International Aviation |  |  |
| ILS | Instrument Landing System |  |  |
| IM | Inner Markers |  |  |
| IM | Intermodulation |  |  |
| IMC | Instrument Meteorological Conditions |  |  |
| IMD | Intermodulation Distortion |  |  |
| IMO | International Maritime Organization |  |  |
| INS | Inertial Navigation System |  |  |
| IRAC | Interdepartment Radio Advisory Committee |  |  |
| ISB | Independent Side Band |  |  |
| ISMLS | Interim Standard Microwave Landing System |  |  |
| ITS | Institute for Telecommunications Sciences |  |  |
| ITT | International Telephone & Telegraph Corporation |  |  |
| ITU | International Telecommunication Union |  |  |
| ITU | D | Telecommunication Development Sector |  |
| ITU | R | Formerly CCIR | ITU Radiocommunications Sector |
| ITU | T | Formerly CCITT | ITU Telecommunication Standards Sector |
| * J * |  |  |  |
| JTIDS | Joint Tactical Information & Distribution System |  |  |
| * K * |  |  |  |
| kHz | Kilohertz |  |  |
| * L * |  |  |  |
| LANT | Atlantic |  |  |
| LAT | Latitude |  |  |
| LF | Low Frequency |  |  |
| LOC | Localizer |  |  |
| LONG | Longitude |  |  |
| LOS | Line | of | sight |
| LSB | Lower Side Band |  |  |
| * M * |  |  |  |
| MA | Mobile Aeronautical (Air/Ground Station Class) |  |  |
| ME | Middle East |  |  |
| MEA | Minimum Enroute IFR Altitude |  |  |
| MET | Meteorological |  |  |
| MF | Middle Frequency/Medium Frequency |  |  |
| MHz | Megahertz |  |  |
| MM | Middle Markers |  |  |
| MO&O | Memorandum Opinion & Order |  |  |
| MOPS | Minimum Operational and Performance Standards |  |  |
| MWARA | Major World Air Route Area |  |  |
| * N * |  |  |  |
| NABER | National Association of Business & Educational Radio, Inc. |  |  |
| NAM | North American (ICAO Air Navigation Region) |  |  |
| NAS | National Airspace System |  |  |
| NAT | North Atlantic (ICAO Air Navigation Region) |  |  |
| NAT | SPG | North Atlantic Systems Planning Group |  |
| NAVAID | Aid to Air Navigation |  |  |
| NBAA | National Business Aviation Association |  |  |
| NBS | National Bureau of Standards |  |  |
| NCA | North Central Asia |  |  |
| NCS | National Communications System |  |  |
| NDB | Non | Directional Radio Beacon |  |
| NDEA | National Defense Emergency Authorization |  |  |
| NEA | North East Asia |  |  |
| NFPRM | Notice of Further Proposed Rule Making |  |  |
| NIAC | National Industry Advisory Committee |  |  |
| NOTAM | Notice to Airmen |  |  |
| NP | North Pacific |  |  |
| NPRM | Notice of Proposed Rule Making |  |  |
| NTIA | National Telecommunications & Information Administration |  |  |
| NWS | National Weather Service |  |  |
| * O * |  |  |  |
| OAC | Oceanic Area Control |  |  |
| OCA | Oceanic Control Area |  |  |
| OFIS | Operational Flight Information Service |  |  |
| OIA | Office of International Affairs |  |  |
| OM | Outside Markers |  |  |
| OPAD | Office of Policy Analysis and Development |  |  |
| OPC | Operational Control |  |  |
| OPS | Operations |  |  |
| OSM | Office of Spectrum Management |  |  |
| OTIA | Office of Telecommunications and Information Applications |  |  |
| * P * |  |  |  |
| PAC | Pacific |  |  |
| PANS | Procedures for Air Navigation Services |  |  |
| PAR | Precision Approach Radar |  |  |
| PLIN | Private Line Intercity Network |  |  |
| PMC | Program Management Committee |  |  |
| PN | Pseudo | Random Noise |  |
| P/P | Point | to | Point |
| PSTN | Public Switched Telephone Network |  |  |
| PTFP | Public Telecommunications Facilities Program |  |  |
| * R * |  |  |  |
| RAC | Rules of the Air & Air Traffic Services (ICAO) |  |  |
| RAPCON | Radar Approach Control |  |  |
| RATCC | Radar Air Traffic Control Center |  |  |
| RC | Ramp Control |  |  |
| RCAG | Remote Center Air/Ground Communications |  |  |
| RCCRCC | Rescue Coordination Center |  |  |
| RDARA | Regional & Domestic Air Route Area |  |  |
| RF | Radio Frequency |  |  |
| RFI | Radio Interference |  |  |
| RGN | Region |  |  |
| RM | Rule Making |  |  |
| RNAV | Area Navigation System |  |  |
| R & O | Report & Order |  |  |
| RSSC | Radio Systems Subcommittee (IATA) |  |  |
| RT | Ramp/Terminal |  |  |
| RTCA | RTCA |  |  |
| RTCM | Radio Technical Commission for Marine |  |  |
| RTR | Remote Transmitter Receiver |  |  |
| RX | Receiver |  |  |
| * S * |  |  |  |
| SA | South Atlantic |  |  |
| SAM | South America |  |  |
| SAM | 1 | South America | West |
| SAM | 2 | South America | East |
| SAM/SAT | South American/South Atlantic |  |  |
| SAR | Search and Rescue |  |  |
| SARDA | State & Regional Disaster Airlift |  |  |
| SARPS | Standards & Recommended Practices (ICAO) |  |  |
| SAT | Satellite |  |  |
| SATCOM | Satellite Communications |  |  |
| SCATANA | Security Control of Air Traffic & Air Navigation Aids |  |  |
| SEA | Southeast Asia (ICAO Air Navigation Region) |  |  |
| SESAR | Single European Sky ATM Research (system) |  |  |
| SELCAL | Selective Calling System |  |  |
| SG | Study Group |  |  |
| SIAC | State Industry Advisory Committee |  |  |
| SIF | Selective Identification Features |  |  |
| SITA | Societe International de Telecommunications Aeronautiques |  |  |
| SMO | Spectrum Management Office (FAA) |  |  |
| SMS | Spectrum Monitoring System |  |  |
| SP | South Pacific |  |  |
| SPG | Systems Planning Group |  |  |
| SSB | Single Side Band |  |  |
| SSR | Secondary Surveillance Radar |  |  |
| SWG | Special Working Group |  |  |
| SWR | Standing Wave Ratio |  |  |
| * T * |  |  |  |
| TA | Type Acceptance |  |  |
| TACAN | Tactical Air Navigation |  |  |
| TAR | Terminal Area Surveillance Radar |  |  |
| TCA | Terminal Control Area |  |  |
| TCAS | Traffic Alert Collision Avoidance System |  |  |
| T/F | Time/Frequency |  |  |
| TMA | Terminal Area Aids |  |  |
| TOP | Technology Opportunities Program |  |  |
| TSA | Transportation Security Administration |  |  |
| TSB | Telecommunications Standardization Bureau |  |  |
| TVOR | Low Power Terminal VOR |  |  |
| TX | Transmitter |  |  |
| * U * |  |  |  |
| UACC | Upper Area Control Center |  |  |
| UAD | Upper Advisory Route |  |  |
| UAR | Upper Air Route |  |  |
| UHF | Ultra | High Frequency |  |
| UFIR | Upper Flight Information Region |  |  |
| USB | Upper Side Band |  |  |
| UTC | Coordinated Universal Time |  |  |
| * V * |  |  |  |
| VASIS | Visual Approach Slope Indicator System |  |  |
| VDL | VHF Digital Link |  |  |
| VFR | Visual Flight Rules |  |  |
| VHF | Very High Frequency |  |  |
| VHF NAV | VHF Navigation |  |  |
| VLF | Very Low Frequency |  |  |
| VLR | Very Low Range |  |  |
| VOLMET | Meteorological Information for Aircraft in Flight |  |  |
| VOR | VHF Omnidirectional Radio Range |  |  |
| VORTAC | Co | located VOR and TACAN |  |
| * W * |  |  |  |
| WAC | World Aeronautical Chart (ICAO) |  |  |
| WARC | World Administrative Radio Conference |  |  |
| WASP | War Air Service Program |  |  |
| WMO | World Meteorological Organization |  |  |
| WRC | World Radio Conference |  |  |
| WX | Weather |  |  |
| * X * |  |  |  |
| XRVHF | Extended Range VHF |  |  |
| * Z * |  |  |  |
| ZI | Zone of Interior |  |  |

TWO-LETTER STATE AND TERRITORY ABBREVIATIONS

AL	-	Alabama				MT	-	Montana

AK	-	Alaska					NB	-	Nebraska

AZ	-	Arizona				NV	-	Nevada

AR	-	Arkansas				NH	-	New Hampshire

CA	-	California				NJ	-	New Jersey

CO	-	Colorado				NM	-	New Mexico

CT	-	Connecticut				NY	-	New York

DE	-	Delaware				NC	-	North Carolina

DC	-	District of Columbia			ND	-	North Dakota

FL	-	Florida					OH	-	Ohio

GA	-	Georgia				OK	-	Oklahoma

GM	-	Gulf of Mexico			OR	-	Oregon

GU	-	Guam					PA	-	Pennsylvania

HI	-	Hawaii					PR	-	Puerto Rico

ID	-	Idaho					RI	-	Rhode Island

IL		-	Illinois					SC	-	South Carolina

IN	-	Indiana					SD	-	South Dakota

IA	-	Iowa					TN	-	Tennessee

KS	-	Kansas					TX	-	Texas

KY	-	Kentucky				UT	-	Utah

LA	-	Louisiana				VA	-	Virginia

ME	-	Maine					VI	-	Virgin Islands

MD	-	Maryland				VT	-	Vermont

MA	-	Massachusetts				WA	-	Washington

MI	-	Michigan				WI	-	Wisconsin

MN	-	Minnesota				WV	-	West Virginia

MS	-	Mississippi				WY	-	Wyoming

MO	-	Missouri

TERMS AND DEFINITIONS

Introduction

For the purposes of these Regulations, the following terms shall have the meaning defined below.  These terms and definitions do not, however, necessarily apply for other purposes.  Definitions identical to those contained in the International Telecommunication Convention (Malaga-Torremolinos, 1973) are marked "(CONV.)".

Administration:  Any governmental department or service responsible for discharging the obligations undertaken in the Convention of the International Telecommunication Union and the Regulations (CONV.)

Aeronautical Earth Station:  An earth station in the fixed-satellite service, or, in some cases, in the aeronautical mobile-satellite service, located at a specified fixed point on land to provide a feeder link for the aeronautical mobile-satellite service.

Aeronautical Station:  A land station in the aeronautical mobile service.  In certain instances, an aeronautical station may be located, for example, on board ship or on a platform at sea.

Aeronautical Fixed Service:  A radiocommunication service between specified fixed points provided primarily for the safety of air navigation and for the regular, efficient and economical operation of air transport.

Aeronautical Mobile-Satellite Service:  A mobile-satellite service in which mobile earth stations are located on board aircraft; survival craft stations and emergency position-indicating radio beacon stations may also participate in this service.

Aeronautical Mobile Service:  A mobile service between aeronautical stations and aircraft stations, or between aircraft stations, in which survival craft stations may participate; emergency position indicating radio beacon stations may also participate in this service on designated distress and emergency frequencies.

Aeronautical Radionavigation-Satellite Service:  A radionavigation-satellite service in which earth stations are located on board aircraft.

Aeronautical Radionavigation Service:  A radionavigation service intended for the benefit and for the safe operation of aircraft.

Aircraft Operator:   An aviation entity that uses ASRI licensed ground radio stations to support their aircraft operations. (Examples: Part 91, 121, and 135 Operators)

Aircraft Station:  A mobile station in the aeronautical mobile service, other than a survival craft station, located on board an aircraft.

Allocation (of a frequency band):  Entry in the Table of Frequency Allocations of a given frequency band for the purpose of its use by one or more terrestrial or space radiocommunication services or the radio astronomy service under specified conditions.  This term shall also be applied to the frequency band concerned.

Allotment (of a radio frequency or radio frequency channel):  Entry of a designated frequency channel in an agreed plan, adopted by a competent conference, for use by one or more administrations for a terrestrial or space radiocommunication service in one or more identified countries or geographical areas and under specified conditions.

Assigned Frequency:  The centre of the frequency band assigned to a station.

Assigned Frequency Band:  The frequency band within which the emission of a station is authorized; the width of the band equals the necessary bandwidth plus twice the absolute value of the frequency tolerance.  Where space stations are concerned, the assigned frequency band includes twice the maximum Doppler shift that may occur in relation to any point of the Earth's surface.

Assignment (of a radio frequency or radio frequency channel):  Authorization given by an administration for a radio station to use a radio frequency or radio frequency channel under specified conditions.

Carrier Power (of a radio transmitter):  The average power supplied to the antenna transmission line by a transmitter during one radio frequency cycle taken under the condition of no modulation.

Class of Emission:  The set of characteristics of an emission, designated by standard symbols, e.g., type of modulation of the main carrier, modulating signal, type of information to be transmitted, and also, if appropriate, any additional signal characteristics.

Coordinated Universal Time (UTC):  Time scale, based on the second (SI), as defined and recommended by the CCIR, and maintained by the International Time Bureau (BIH).  For most practical purposes associated with the Radio Regulations, UTC is equivalent to mean solar time at the prime meridian (0o longitude), formerly expressed in GMT.

Effective Radiated Power (e.r.p.) (in a given direction):  The product of the power supplied to the antenna and its gain relative to a half-wave dipole in a given direction.

Emission:  Radiation produced, or the production of radiation, by a radio transmitting station.  For example, the energy radiated by the local oscillator of a radio receiver would not be an emission but a radiation.

Equivalent Isotopically Radiated Power (e.i.r.p.):  The product of the power supplied to the antenna and the antenna gain in a given direction relative to an isotropic antenna (absolute or isotropic gain).

Frequency Tolerance:  The maximum permissible departure by the centre frequency of the frequency band occupied by an emission from the assigned frequency or, by the characteristic frequency of an emission from the reference frequency.  The frequency tolerance is expressed in parts in 106 or in hertz.

Full Carrier Single-Sideband Emission:  A single-sideband emission without reduction of the carrier.

Gain of an Antenna:  The ratio, usually expressed in decibels, of the power required at the input of a loss-free reference antenna to the power supplied to the input of the given antenna to produce, in a given direction, the same field strength or the same power flux-density at the same distance.  When not specified otherwise, the gain refers to the direction of maximum radiation.  The gain may be considered for a specified polarization.

Depending on the choice of the reference antenna a distinction is made between:

a.	absolute or isotropic gain (Gi), when the reference antenna is an isotropic antenna isolated in space;

b.	gain relative to a half-wave dipole (Gd), when the reference antenna is a half-wave dipole isolated in space whose equatorial plane contains the given direction;

c.	gain relative to a short vertical antenna (Gv), when the reference antenna is a linear conductor, must be shorter than one quarter of a wavelength, normal to the surface of a perfectly conducting plane which contains the given direction.

Harmful Interference*:  Interference which endangers the functioning of a radionavigation service or of other safety services or seriously degrades, obstructs, or repeatedly interrupts a radiocommunication service operating in accordance with these Regulations.

Industrial, Scientific and Medical (ISM) Applications (of radio frequency energy):  Operation of equipment or appliances designed to generate and use locally radio frequency energy for industrial, scientific, medical, domestic or similar purposes, excluding applications in the field of telecommunications.

Interference:  The effect of unwanted energy due to one or a combination of emissions, radiations, or inductions upon reception in a radiocommunication system, manifested by any performance degradation, misinterpretation, or loss of information which could be extracted in the absence of such unwanted energy.

Mean Power (of a radio transmitter):  The average power supplied to the antenna transmission line by a transmitter during an interval of time sufficiently long compared with the lowest frequency encountered in the modulation taken under normal operating conditions.

Mobile Service:  A radiocommunication service between mobile and land stations, or between mobile stations (CONV.).

Movement Area: The runways, taxiways, and other areas on an airport/heliport which are utilized for taxiing/hover taxiing, air taxiing, takeoff, and landing aircraft, exclusive of loading ramps and parking areas. At those airports/heliports with a tower, specific approval for entry onto the movement area must be obtained from ATC. (See ICAO term for Movement Area.)

Movement Area (ICAO): That part of an aerodrome to be used for the takeoff, landing and taxiing of aircraft, consisting of the maneuvering area and the apron(s).

Necessary Bandwidth: For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

Non-Aircraft Operator: A business entity that uses ASRI licensed ground radio stations to support their customers’ aircraft operations. (Examples: Million Air, AAR Oklahoma, BAX Global, etc.)

Non-Movement Area: Taxiways and apron (ramp) areas not under the control of air traffic.

Occupied Bandwidth: The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage B/2 of the total mean power of a given emission.

Unless otherwise specified by the CCIR for the appropriate class of emission, the value of B/2 should be taken as 0.5%.

Out-of-Band Emission*:  Emission on a frequency or frequencies immediately outside the necessary bandwidth which results from the modulation process, but excluding spurious emissions.

Peak Envelope Power (of a radio transmitter):  The average power supplied to the antenna transmission line by a transmitter during one radio frequency cycle at the crest of the modulation envelope taken under normal operating conditions.

Power:  Whenever the power of the radio transmitter etc. is referred to it shall be expressed in one of the following forms, according to the class of emission, using the arbitrary symbols indicated:

- peak envelope power (PX or pX);		- mean power (PY or pY);

- carrier power (PZ or pZ).

For different classes of emission, the relationships between peak envelope power, mean power and carrier power, under the conditions of normal operation and of no modulation, are contained in CCIR Recommendations which may be used as a guide.

For use in formulae, the symbol p denotes power expressed in watts and the symbol P denotes power expressed in decibels relative to a reference level.

Public Correspondence:  Any telecommunication which the offices and stations must, by reason of their being at the disposal of the public, accept for transmission (CONV.).

Radiation:  The outward flow of energy from any source in the form of radio waves.

Radiocommunication:  Telecommunication by means of radio waves (CONV.).

Radiodetermination:  The determination of the position, velocity and/or other characteristics of an object, or the obtaining of information relating to these parameters, by means of the propagation properties of radio waves.

Radio Direction-Finding:  Radiodetermination using the reception of radio waves for the purpose of determining the direction of a station or object.

Radiolocation:  Radiodetermination used for purposes other than those of radionavigation.

Radionavigation:  Radiodetermination used for the purposes of navigation, including obstruction warning.

Reduced Carrier Single Sideband Emission:  A single-sideband emission in which the degree of carrier suppression enables the carrier to be reconstituted and to be used for demodulation.

Reference Frequency:  A frequency having a fixed and specified position with respect to the assigned frequency.  The displacement of this frequency with respect to the assigned frequency has the same absolute value and sign that the displacement of the characteristic frequency has with respect to the centre of the frequency band occupied by the emission.

Safety Service: Any radiocommunication service used permanently or temporarily for the safeguarding of human life and property (CONV.).

Service Provider:  A business entity that uses ASRI licensed ground radio stations to provide voice and/or data link communications services to their customers. (Examples: SITA, ARINC, etc.)

Single-Sideband Emission:  An amplitude modulated emission with one sideband only.

Spurious Emission*: Emission on a frequency or frequencies which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information.  Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products, but exclude out-of-band emissions.

Station: One or more transmitters or receivers or a combination of transmitters and receivers, including the accessory equipment, necessary at one location for carrying on a radiocommunication service, or the radio astronomy service.

Each station shall be classified by the service in which it operates permanently or temporarily.

Suppressed Carrier Single-Sideband Emission:  A single-sideband emission in which the carrier is virtually suppressed and not intended to be used for demodulation.

Survival Craft Stations: A mobile station in the maritime mobile service or the aeronautical mobile service intended solely for survival purposes and located on any lifeboat, life-raft or other survival equipment.

Telecommunication: Any transmission, emission or reception of signs, signals, writing, images and sounds or intelligence of any nature by wire, radio, optical or other electromagnetic systems (CONV.).

Unwanted Emissions*:  Consist of spurious emissions and out-of-band emissions.

* NOTE BY THE GENERAL SECRETARIAT:  THIS DEFINITION IS NOT IN ALIGNMENT WITH ANNEX 2 TO THE CONVENTION.  THE CORRESPONDING DEFINITION IN THAT ANNEX SHALL PREVAIL TO THE EXTENT THAT THERE ARE DIFFERENCES BETWEEN THEM (SEE ALSO RESOLUTION 68).

# AFC Composition and Member Lists

## Past Chairman

PAST CHAIRMAN OF THE AERONAUTICAL FREQUENCY COMMITTEE

| Name | Year | Organization |
| --- | --- | --- |
| C. A. Petry | 1958 | XA |
| H. L. Garrison | 1959 | UA |
| D. R. Buckler | 1960 | EA |
| J. M. Reilly | 1961 | AA |
| H. O. Harrison | 1962 | BN |
| E. G. Jones | 1963 | DL |
| V. N. Williams | 1964 | PA |
| D. R. Buckler | 1965 | EA |
| R. J. Glischinski | 1966 | NW |
| L. S. Gallemore | 1967 | TWA |
| K. J. Rhead | 1968 | UA |
| T. J. Sanders | 1969 | AA |
| J. S. Glenn | 1970 | RW |
| R. M. Bennett | 1971 | FT |
| S. A. Moore | 1972 | BN |
| A. R. Clark | 1973 | CO |
| K. M. Ross | 1974 | EA |
| G. R. Carter | 1975 | AA |
| H. H. Fink | 1976 | DL |
| L. S. Gallemore | 1977 | TW |
| M. O. Everhart | 1978 | PI |
| G. A. Carlson | 1979 | UA |
| L. S. Donahue | 1980 | AC |
| R. M. Bennett | 1981 | FT |
| W. H. Stine, II | 1982 | NBAA |
| C. Dennis Wright | 1983 | AOPA |
| W. Martin | 1984 | DL |
| H. Brown | 1985 | AA |
| H. Smith | 1986 | TW |
| G. E. Allen | 1987 | EAL |
| G. E. Allen | 1988 | EAL |
| W. H. Stine, II | 1989 | NBAA |
| P. Anderson | 1990 | NWA |
| D. Tangney | 1991 | UAL |
| B. Funk | 1992 | AOPA |
| H. R. May | 1993 | AAL |
| D. Riddle | 1994 | USA |
| Tim Totten | 1995 | UPS |
| Chris Bogg | 1996 | AWE |
| H. Robert May | 1997 | AAL |
| P. Anderson | 1998 | NWA |
| T. King | 1999 | DAL |
| M. Swaringen | 2000 | USA |
| Bill Stine | 2001 | NBAA |
| Tim Totten | 2002 | UPS |
| Nathan Lemon | 2003 | FDX |
| Bill Winfrey | 2004 | DAL |
| Chris Kelly | 2005 | COA |
| Robert Lee | 2006 | UAL |
| Rich Farr | 2007 | AAL |
| Bill Stine | 2008 | NBAA |
| Pete Talbot | 2009 | HSAC |
| Paul Anderson | 2010 | NWA/DAL |
| John Gana | 2011 | COA/UAL |
| Neal Young | 2012 | SWA |
| Vytas Cerniauskas | 2013 | ACG/ASA |
| Tim Totten | 2014 | UPS |
| Terry Gambill | 2015 | PHI |
| Vytas Cerniauskas | 2016 | ACG/ASA |

## The ASRI Board of Directors

The following companies make up the Board of Directors of Aviation Spectrum Resources, Inc. as of 25 May, 2016:

- American Airlines, Inc.

- Aviation Spectrum Resources, Inc.

- Delta Airlines, Inc.

- United Airlines, Inc.

- Independent Member (Chairman)

NOTE: For additional information concerning the ASRI board members, contact ASRI management personnel at 410-266-4800 or info@asri.aero

## Members, Associates, and Staff

### AFC Membership Listing

Due to the continuous change in the AFC membership, this information is only available on the ASRI web site (www.asri.aero) under the “AFC Members Only” section.  If you cannot access or view this list on the ASRI web site, then contact ASRI at 410-266-4800 or info@asri.aero for assistance.

### Honorary Members

The following are AFC members that the committee has designated as permanent lifetime honorary members in recognition of their outstanding service to the committee and the entire aviation industry.

E. G. Jones (DAL) - June 28, 1979

Harold H. Fink (DAL) - April 15, 1980

K. A. Moore (COA) - August 27, 1980

Lee S. Donahue (AC) - September 1, 1981

Lorrence S. Gallemore (TWA) - December 16, 1982

H. S. Smith (XA) - February 27, 1985

Mitch Everhart (PAI) - December 31, 1986

Harvey L. Brown (AAL) - December 31, 1986

Larry Berryhill (UAL) - December 31, 1986

Wayland L. Martin (DAL) - March 31, 1989

Donald Trombley (ATA) - May 19, 1993

Dick Covell (ARINC) - August 26, 1993

Edward Adelson (ARINC) - February 22, 2001

H. Robert May (AAL) - June 27, 2001

U. W. King (DAL) - October 31, 2003

Chris Bogg (USA) – October 11, 2011

Brian Gaffney (ARINC) – April 25, 2013

Richard Stutz (ASRI) – June 18, 2013

Bill Stine (NBAA) – December 2, 2013

Valid only for the meeting they are elected.

An organization with AFC membership is only eligible for 1 vote, no matter how many members are in attendance from the same organization

A single attendee to a meeting can hold multiple proxies if nominated by different AFC Full Members.

Except for previously approved AFC meeting minutes.  See section 1.4.8.

Either intentionally, or unintentionally.

The FCC Third Report and Order in WT Docket No. 01-289 dated June 1, 2010, amended Part 87 of the Commission’s Rules to permit the use of 8.33 kHz channel spacing in the aeronautical enroute service and by flight test stations.

This distance is that at which the FAA recommends at least 500 kHz separation of adjacent signals, and is approximately that required to prevent adjacent channel interference as in the cosite case, but with a receiver adjacent channel rejection of 100 dB.

This distance is approximately the separation required to prevent adjacent channel interference in a receiver having 60 dB adjacent channel rejection, and 2uV squelch threshold, when subject to a 10 watt transmitter on an adjacent channel.

Ground personnel refers to the Operations, Maintenance, Ramp Parking, Ground Handling and other personnel charged with servicing the aircraft.

47 CFR Part 87.261(b)

All coordination altitudes are relative to mean sea level, except for ramp/terminal (RT) and helicopter (HO), which are referenced to ground level.

Annex 10 to the ICAO Convention, Volume 1 Part II, Ch. 1

As required by the Federal Air Regulations for U.S. operations certificated under 14 CFR Part 121.

47 CFR Part 87.261(a)

47 CFR Part 87.265

Although some air carriers have agreed to perform certain ramp control communications in "non-movement areas," (as defined by the FAA) there is no agreement as to whether this is an ATS or AOC function.

Paying ASRI to maintain a single license on a continuous basis is considered formal notification that the license is in use and required by that organization.  However, ASRI may investigate single frequency users to ensure they are operating as licensed.

A frequency is considered to be underutilized when its loading does not exceed 10% as measured in accordance with Section 2.7.2.

The system operator is encouraged to monitor all system frequencies within line-of-sight of the test point to support determination of the most appropriate frequency.

ARINC comment:  Due to hidden terminals, enroute performance degrades from CSMA performance to near aloha performance.  Peak throughput on aloha channels is 18% of presented load and occurs when the normalized presented load is 50%.  The RF utilization at this peak is 39.4%. It is conceivable that the technical justification of the original voice channel requirement of 40% came from this value of 39.4%.  It should be noted that as channel loading approaches this peak, delays become long, which may be acceptable for AOC traffic.  On the other hand, ATC traffic usually has delay requirements which would preclude using an enroute channel operating anywhere near this peak load.  The Aloha model may be overly pessimistic for VDL due to factors such as the capture performance of the avionics, but this has never been validated in the field.

SITA comment:  The performance of a VDL Mode 2 data link channel has already been analyzed in great detail through simulation by various organizations. The simulation scenerios for VDL Mode 2 channel operations at different traffic levels supported both a small terminal area as well as a wider en-route area. The most demanding case was for an en-route VDL Mode 2 channel providing service to 150-200 aircraft. The results indicated that as long as the channel utilization was below 50%, the performance requirements were met consistently.

SITA comment:  It should be noted that the simulation results also indicated that when a VDL Mode 2 channel is restricted to providing data link services in a terminal area only (aircraft at the airport) the performance requirements are met even when the channel is utilized as much as 75%. However, this criteria cannot be applied to the one VDL Mode 2 channel in use today, that is the Common Signaling Channel (CSC), because the CSC is used for services in terminal areas as well as enroute. Since the enroute usage is more sensitive to channel loading, due to CSMA inefficiencies such as hidden transmitter for example, the more stringent enroute metrics must be used.

In order to ensure that each CSP has sufficient spectrum to maintain adequate network performance to support ATS operations while maintaining efficient spectrum usage, the following spectrum request criteria are used.  The Data Comm message latency requirement is the principal driver for additional VDL-2 spectrum.   The message latency statistic employed as a metric for spectrum request is the 95% message latency for each service volume.  When the 95% message latency breaches 9 seconds in any given service volume, the CSP is out of compliance for that service volume and air traffic operations are affected.  The spectrum request criteria permit the CSPs to procure spectrum and deploy appropriate hardware before message latency exceeds the Data Comm requirement, while ensuring efficient use of the limited spectrum.

Attachment 1, to Letter of Agreement, Andre Lapointe, Acting Deputy Minister of Communications, Canada to Charles D. Ferris, Chairman, Federal Communications Commission, dated December 20, 1977.

21 “Treaties and Other International Acts” Series 5205; An agreement between the U.S. and Canada dated October 24, 1962, revised by letter adopted by the FCC on September 14, 1977.

22 All coordination altitudes are relative to mean sea level, except for ramp/terminal (RT), which is the local ground level.

47 CFR § 2.106 - US Footnote 244: The band 136–137 MHz is allocated to the non-Federal aeronautical mobile (R) service on a primary basis, and is subject to pertinent international treaties and agreements. The frequencies 136, 136.025, 136.05, 136.075, 136.1, 136.125, 136.15, 136.175, 136.2, 136.225, 136.25, 136.275, 136.3, 136.325, 136.35, 136.375, 136.4, 136.425, 136.45, and 136.475 MHz are available on a shared basis to the Federal Aviation Administration for air traffic control purposes, such as automatic weather observation stations (AWOS), automatic terminal information services (ATIS), flight information services-broadcast (FIS-B), and airport control tower communications, Dated 10 October 2010.

ASRI will make every effort to avoid interference in the planning of assignments.  However, this is not always possible, and new licensees accept that the interference environment may require additional measure to ensure effective radio operation.